Cygnus Data Systems'

���     ��   ��   ������   ��   ��   ������   �������

��   ��   ���  ��   ��       ��   ��   ��       ��   ��

�������   �� � ��   ������   ��   ��   ����     �� ���

��   ��   ��  ���       ��   �� � ��   ��       ��   ��

��   ��   ��   ��   ������    �� ��    ������   ��   ��

Copyright (C) 1989-94 by Cygnus Data Systems

Version 3.7     Release 05/15/94

The ANSWER.exe

Voice Mail and Telephone Answering System

User's Reference Manual

Printing 05/15/94

_______________________________________________________

ANSWER Voice Mail System                Contents

Table of Contents

1. Preliminary  . . . . . . . . . . . . . . . . . . . . . . .   4

1.1  General . . . . . . . . . . . . . . . . . . . . . . .   4

1.2  Distribution  . . . . . . . . . . . . . . . . . . . .   4

1.3  License . . . . . . . . . . . . . . . . . . . . . . .   5

1.4  Contacting the Author . . . . . . . . . . . . . . . .   9

1.5  Acknowledgements  . . . . . . . . . . . . . . . . . .   9

2. Requirements . . . . . . . . . . . . . . . . . . . . . .    10

3. Introduction . . . . . . . . . . . . . . . . . . . . . .    11

4. Setup  . . . . . . . . . . . . . . . . . . . . . . . . .    12

4.1  Hardware Installation . . . . . . . . . . . . . . .    12

4.1.1  Modems . . . . . . . . . . . . . . . . . . .    12

4.1.1.1  Addresses . . . . . . . . . . . . .    13

4.1.1.2  Interrupts  . . . . . . . . . . . .    13

4.1.2  The DigiPhone  . . . . . . . . . . . . . . .    13

4.1.2.1  Interrupts  . . . . . . . . . . . .    13

4.1.2.2  Addresses . . . . . . . . . . . . .    14

4.2  Software Installation . . . . . . . . . . . . . . .    16

4.2.1  Installing The Software  . . . . . . . . . .    16

4.2.2.1  First Run . . . . . . . . . . . . .    16

4.2.2.2  Answering Machine . . . . . . . . .    17

4.2.2.3  Voice Mail Box  . . . . . . . . . .    18

4.2.2.4  Information Server  . . . . . . . .    19

4.2.2.5  PhoneBook Dialer  . . . . . . . . .    19

5. Operation  . . . . . . . . . . . . . . . . . . . . . . .    20

5.1  Phone . . . . . . . . . . . . . . . . . . . . . . .    20

5.1.1  On Hook  . . . . . . . . . . . . . . . . . .    20

5.1.2  Off Hook . . . . . . . . . . . . . . . . . .    20

5.1.3  Dial . . . . . . . . . . . . . . . . . . . .    20

5.1.4  Flash  . . . . . . . . . . . . . . . . . . .    20

5.1.5  Terminal Mode  . . . . . . . . . . . . . . .    20

5.2  Messages  . . . . . . . . . . . . . . . . . . . . .    21

5.2.1  Play . . . . . . . . . . . . . . . . . . . .    21

5.2.2  Record . . . . . . . . . . . . . . . . . . .    21

5.2.3  InComing . . . . . . . . . . . . . . . . . .    21

5.2.3.1  Play  . . . . . . . . . . . . . . .    21

5.2.3.2  Call Log  . . . . . . . . . . . . .    21

5.2.3.3  Times/Dates . . . . . . . . . . . .    22

5.2.3.4  Delete  . . . . . . . . . . . . . .    22

5.2.3.5  Record  . . . . . . . . . . . . . .    22

5.2.4  OutGoing . . . . . . . . . . . . . . . . . .    22

5.2.4.1  Record  . . . . . . . . . . . . . .    22

5.2.4.2  Select  . . . . . . . . . . . . . .    22

5.2.4.3  Edit  . . . . . . . . . . . . . . .    23

5.2.4.4  Delete  . . . . . . . . . . . . . .    23

5.2.4.5  Play  . . . . . . . . . . . . . . .    23

5.2.5  Examine  . . . . . . . . . . . . . . . . . .    23

5.2.5.1  Graphic Editor  . . . . . . . . . .    23

5.2.5.2  Spectrum Analyzer . . . . . . . . .    23

5.2.5.3  Diagnostics . . . . . . . . . . . .    24

5.3  Activate  . . . . . . . . . . . . . . . . . . . . .    24

1

ANSWER Voice Mail System                Contents

5.3.1  Answering Machine  . . . . . . . . . . . . .    24

5.3.1.1  Wait for Calls  . . . . . . . . . .    24

5.3.1.2  Answering the Phone . . . . . . . .    25

5.3.1.3  Outgoing Messages Delivery  . . . .    25

5.3.1.4  Listen for Touch-Tone . . . . . . .    25

5.3.1.5  Page Operator . . . . . . . . . . .    25

5.3.1.6  Fax Code Errorlevel Exit  . . . . .    25

5.3.1.7  Remote Mode . . . . . . . . . . . .    26

5.3.1.8  Incoming Message Recording  . . . .    26

5.3.1.9  Session Completion  . . . . . . . .    26

5.3.2  Voice Mail Box . . . . . . . . . . . . . . .    26

5.3.2.1  Wait for Calls  . . . . . . . . . .    26

5.3.2.2  Answering the Phone . . . . . . . .    26

5.3.2.3  Mail Box Number . . . . . . . . . .    27

5.3.2.4  Edit Extension  . . . . . . . . . .    27

5.3.3  Information Server . . . . . . . . . . . . .    27

5.3.4  PhoneBook Dialer . . . . . . . . . . . . . .    27

5.3.4.1  PhoneBook . . . . . . . . . . . . .    28

5.3.4.2  FileList  . . . . . . . . . . . . .    28

5.3.4.3  Manual  . . . . . . . . . . . . . .    28

5.3.4.3.1  Outgoing Calls  . . . . . . . . .    28

5.4  Control . . . . . . . . . . . . . . . . . . . . . .    28

5.4.1  Answering Machine  . . . . . . . . . . . . .    28

5.4.1.1  OGM Number  . . . . . . . . . . . .    28

5.4.1.2  Rings . . . . . . . . . . . . . . .    29

5.4.1.3  Remote Code . . . . . . . . . . . .    29

5.4.1.4  Answer Mode . . . . . . . . . . . .    29

5.4.2  Voice Mail Box . . . . . . . . . . . . . . .    29

5.4.2.1  Extension Digits  . . . . . . . . .    29

5.4.2.2  Password Digits . . . . . . . . . .    30

5.4.2.3  Edit Extension No.  . . . . . . . .    30

5.4.2.4  Incoming Msgs . . . . . . . . . . .    30

5.4.2.5  User/Extension Edit . . . . . . . .    30

5.4.3  Information Server . . . . . . . . . . . . .    30

5.4.3.1  Edit Menu . . . . . . . . . . . . .    30

5.4.3.2  TED Editor  . . . . . . . . . . . .    33

5.4.3.3  Other Editor  . . . . . . . . . . .    34

5.4.4  PhoneBook Dialer . . . . . . . . . . . . . .    34

5.4.4.1  Type  . . . . . . . . . . . . . . .    34

5.4.4.2  Name  . . . . . . . . . . . . . . .    34

5.4.4.3  Code  . . . . . . . . . . . . . . .    34

5.4.4.4  Number  . . . . . . . . . . . . . .    34

5.4.4.5  Script and Last Called  . . . . . .    34

5.4.4.6  Dial Options  . . . . . . . . . . .    34

5.5  Help  . . . . . . . . . . . . . . . . . . . . . . .    35

5.5.1  Instant Help . . . . . . . . . . . . . . . .    35

5.6  Options . . . . . . . . . . . . . . . . . . . . . .    36

5.6.1  Colors . . . . . . . . . . . . . . . . . . .    36

5.6.2  Directory  . . . . . . . . . . . . . . . . .    36

5.6.2.1  Program . . . . . . . . . . . . . .    36

5.6.2.2  InComing  . . . . . . . . . . . . .    36

5.6.2.3  OutGoing  . . . . . . . . . . . . .    36

5.6.2.4  Temporary . . . . . . . . . . . . .    36

5.6.2.5  Bin . . . . . . . . . . . . . . . .    36

5.6.3  Hardware   . . . . . . . . . . . . . . . . .    36

5.6.3.1  Addresses . . . . . . . . . . . . .    36

2

ANSWER Voice Mail System                Contents

5.6.3.2  Mouse . . . . . . . . . . . . . . .    36

5.6.3.3  Zero Level  . . . . . . . . . . . .    37

5.6.4  Messages . . . . . . . . . . . . . . . . . .    37

5.6.4.1  Limits  . . . . . . . . . . . . . .    37

5.6.4.2  Buffers . . . . . . . . . . . . . .    37

5.6.4.3  Copy  . . . . . . . . . . . . . . .    37

5.6.4.4  Support . . . . . . . . . . . . . .    37

5.6.4.4.1NUMS  . . . . . . . . . . . . . . .    37

5.6.4.4.2DAYS  . . . . . . . . . . . . . . .    37

5.6.4.4.3MONTHS  . . . . . . . . . . . . . .    38

5.6.5  Tones  . . . . . . . . . . . . . . . . . . .    38

6. Voice Message Files  . . . . . . . . . . . . . . . . . .    39

6.1  Formats . . . . . . . . . . . . . . . . . . . . . .    39

6.1.1  DigiPhone voice file format  . . . . . . . .    39

6.1.2  CompuCom voice file format . . . . . . . . .    39

6.1.3  ZyXEL voice file format  . . . . . . . . . .    39

6.1.4  ZOOM voice file format . . . . . . . . . . .    39

6.2.3  VOC Files  . . . . . . . . . . . . . . . . .    39

6.3.2  WAV Files  . . . . . . . . . . . . . . . . .    40

6.2  Conversions . . . . . . . . . . . . . . . . . . . .    40

6.2.1  VCNVT.EXE  . . . . . . . . . . . . . . . . .    40

6.2.2  ZYX2WAV.EXE  . . . . . . . . . . . . . . . .    40

6.2.3  ZYXELVOC.EXE . . . . . . . . . . . . . . . .    41

6.2.4  VOC2WAV.EXE  . . . . . . . . . . . . . . . .    41

6.2.5  WAV2VOC.EXE  . . . . . . . . . . . . . . . .    41

6.3  Standalone Utilities  . . . . . . . . . . . . . . .    41

6.3.1  ANSPlay  . . . . . . . . . . . . . . . . . .    42

6.3.2  ANSRec . . . . . . . . . . . . . . . . . . .    42

6.3.3  Receive Fax  . . . . . . . . . . . . . . . .    42

7. Programming Examples . . . . . . . . . . . . . . . . . .    42

8. Files  . . . . . . . . . . . . . . . . . . . . . . . . .    43

9. Command Line Parameters  . . . . . . . . . . . . . . . .    45

10.     Return Codes  . . . . . . . . . . . . . . . . . . .    46

11.     Index . . . . . . . . . . . . . . . . . . . . . . .    47

3

ANSWER Voice Mail System 1 / Preliminary                        4

1.   Preliminary

Information in this document is subject to change without notice

and does not represent a commitment on the part of Cygnus Data

Systems.  The software described in this document is furnished

under a license agreement or a nondisclosure agreement.  The

software may be used or copied only in accordance with the terms

of the agreement.

1.1  General

ANSWER.EXE v3.6 was the first shareware release of this software.

It has previously remained distributed in the commercial market.

ANSWER is user supported software and represents hundreds of

hours of my personal programming time over the past 5 years.

Originally designed for the DigiPhone Voice Mail Card senior

project, it has since grown to support other voice mail cards,

and I will continue to add support for additional voice mail

cards in the future.

1.2  Distribution

This software is also distributed under the ShareWare concept.

You may use this software for the purposes of evaluation for a

period of 30 days.  After that period, if you continue to use the

software, you must register it.

If you received this software under the shareware distribution

method, you are strongly encouraged to register the software and

support its continuing development.  Although it has extensive

features, I have a long list of features I would like to add to

it.  However, without the support of its users, I will be unable

to do this.  Please consider registering it and becoming one of

the many appreciated supporters.

No part of this manual or software may be reproduced,

transmitted, transcribed, stored in a retrieval system, or

translated into any language, in any form or by any means, except

as provided for in the following license without the prior

written consent of Cygnus Data Systems.

Permission is hereby granted to copy, distribute, and use this

manual and the described software provided the following

conditions are met:  You may make exact copies of the authorized

distribution archives and distribute them, as long as the

original files remain intact and unaltered.  You may upload them

to Bulletin Board Systems.  No charge may be made for the

distribution of this manual and software, this particularly

applies to disk vendors and electronic bulletin board systems

which are run for a profit basis, except for a maximum of $3

charge for the disk media used to transfer the software.  No

changes or modifications may be made to either the software or

the manual.  No attempt may be made to decompile, disassemble, or

otherwise reverse engineer the software.  This software and

manual may not be rented or leased to others.  Commercial,

corporate, and governmental use is allowed if and only if the

software and manual are purchased on a one package per CPU basis.

ANSWER Voice Mail System 1 / Preliminary                        5

1.3  License

The ANSWER.EXE Voice Mail System software is

Copyright (C) 1989 - 1994 by Cygnus Data Systems.

End User Software License Agreement.

1.3.1. Software.  ANSWER.EXE Voice Mail System is comprised of

the computer programs and documentation contained in this package

and is referred to in this license agreement as the "Software"

and in the documentation as "ANSWER".

1.3.2. A "Cygnus Data Systems Product".  The Software is a

product owned and developed by Cygnus Data Systems.

1.3.3. Licensee.  Licensee is the person or entity who purchased

this License or for whom this License was purchased.  The

development location shall be considered as those sites which

belong to the entity at which software development takes place.

The CPU system shall be considered the computing hardware on

which the software is run.

1.3.4. License.  Cygnus Data Systems sells and grants to

Licensee, and Licensee purchases, a nonexclusive, nontransferable

license (the "License") to use the Software in accordance with

this Agreement at a single CPU system running under PC-DOS or

MS-DOS operating system.  No two CPU systems may run one

registered copy of the software at the same time.  Rights not

expressly granted are reserved by Cygnus Data Systems.

1.3.5. Distribution Rights.  The Software includes "Linkable

Routines" and "Distributable Files."  Linkable Routines consist

of the object code routines in the Software (i.e. *.obj created

from *.h, *.c, and *.asm).  The License includes the right to

incorporate Linkable Routines into software applications

developed by Licensee and to distribute them as part of such

applications, provided that the Linkable Routines have been

incorporated in such a way that they cannot be separated out of

the applications.  Distributable Files consist of those

"run-time" files identified in the Software documentation as

required during execution of Licensee's applications.  This

License further includes the right to distribute Distributable

Files as part of the software applications developed by Licensee.

No other portions of the Software may be distributed or

transferred by Licensee.  THE LINKABLE ROUTINES AND DISTRIBUTABLE

FILES MAY NOT BE DISTRIBUTED WITH, OR AS PART OF, ANY LIBRARY

TYPE PRODUCT, SOFTWARE DEVELOPMENT TOOL OR OPERATING SYSTEM,

WITHOUT THE EXPRESS WRITTEN PERMISSION OF CYGNUS DATA SYSTEMS.

1.3.6.  Copies and Adaptations.  Licensee may make or authorize

the making of copies or adaptations of the Software provided that

any such new copy or adaptation: (i) is created as an essential

step in the utilization of the Software at the licensed

development location and is used in no other manner, or (ii) is

only for archival purposes to back-up the licensed location.  All

trademark and copyright notices must be faithfully reproduced and

included on such copies and adaptations. Notwithstanding the

foregoing, Licensee may not copy the Software documentation.

ANSWER Voice Mail System 1 / Preliminary                        6

1.3.7.  Source Code.  The License is limited to the Software in

machine executable code and object code form only and does not

include any rights to source code unless a separate Source Code

License is purchased.  (See source code agreement).

1.3.8.  Protection of Software.  Except as expressly authorized

in this Agreement, Licensee may not: (i) disassemble, decompile,

or otherwise reverse engineer the Software, or (ii) create

derivative works based upon the Software, or (iii) rent, lease,

sublicense, distribute, transfer, copy, reproduce, modify or

timeshare the Software, or (iv) allow any third party to access

or use the Software, or (v) modify the Software (including any

deletion of code from or addition of code to the Software).

1.3.9.  Ownership.  Ownership of, and title to, the Software

(including any adaptations) and all copies thereof (including

copies provided with this Agreement and copies under Paragraph 6)

shall be held by Cygnus Data Systems.  Copies are loaned to

Licensee to allow Licensee to exercise rights under the License.

Only the License is purchased by Licensee.

1.3.10.  Transfer of License.  Licensee may transfer the License

to another person or entity if Licensee first delivers to Cygnus

Data Systems the written agreement of the transferee to be bound

by this Agreement.  If the License is transferred, Licensee must

permanently transfer to the transferee all copies of the Software

including the original copies provided in this package and copies

under Paragraph 6.

1.3.11.  Limited Warranty.

1.3.11(a).  Registration Card.  Licensee shall have no rights

under this Limited Warranty unless the enclosed Registration Card

is promptly completed, signed, and returned to Cygnus Data

Systems, or unless the Licensee receives and Invoice from Cygnus

Data Systems.

1.3.11(b).  Diskettes and Documentation.  Cygnus Data Systems

warrants that if the enclosed magnetic diskettes or documentation

are in a damaged or physically defective condition at the time

that the License is purchased and if they are returned to Cygnus

Data Systems (postage prepaid) within 90 days of the purchase,

then Cygnus Data Systems will provide the Licensee with

replacements at no charge.

1.3.11(c)  Software.  Cygnus Data Systems warrants that if the

Software fails to substantially conform to the specifications of

the Software documentation or to any other Software

specifications published by Cygnus Data Systems and if the

nonconformity is reported in writing by Licensee to Cygnus Data

Systems within 90 days from the date the License is purchased,

then Cygnus Data Systems shall either remedy the nonconformity or

offer to refund the purchase price to Licensee upon a return of

all copies of the Software to Cygnus Data Systems.  In the event

of a refund the License shall terminate.

ANSWER Voice Mail System 1 / Preliminary                        7

1.3.12.  Disclaimer and Limitations.

1.3.12(a).  DISCLAIMER OF WARRANTIES.  CYGNUS DATA SYSTEMS MAKES

NO WARRANTY, REPRESENTATION OR PROMISE NOT EXPRESSLY SET FORTH IN

THIS AGREEMENT.  CYGNUS DATA SYSTEMS DISCLAIMS AND EXCLUDES ANY

AND ALL IMPLIES WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A

PARTICULAR PURPOSE.  CYGNUS DATA SYSTEMS DOES NOT WARRANT THAT

THE SOFTWARE WILL SATISFY LICENSEE'S REQUIREMENTS OR THAT THE

SOFTWARE IS WITHOUT DEFECT OR ERROR OR THAT THE OPERATION OF THE

SOFTWARE WILL BE UNINTERRUPTED.

1.3.12(b).  LIMITATION ON LIABILITY.  THE AGGREGATE LIABILITY OF

CYGNUS DATA SYSTEMS ARISING FROM OR RELATING TO THIS AGREEMENT OR

THE SOFTWARE (REGARDLESS OF THE FORM OF ACTION OR CLAIM - E.G.,

CONTRACT, WARRANTY, TORT, COMPUTER MALPRACTICE, FRAUD AND/OR

OTHERWISE) IS LIMITED TO THE TOTAL OF ALL PAYMENTS MADE BY OR FOR

LICENSEE TO PURCHASE THE LICENSE.  CYGNUS DATA SYSTEMS SHALL NOT

IN ANY CASE BE LIABLE FOR ANY SPECIAL, INCIDENTAL, CONSEQUENTIAL,

INDIRECT OR PUNITIVE DAMAGES EVEN IF CYGNUS DATA SYSTEMS HAS BEEN

ADVISED OF THE POSSIBILITY OF SUCH DAMAGES. CYGNUS DATA SYSTEMS

IS NOT RESPONSIBLE FOR LOST PROFITS OR REVENUE, LOSS OF USE OF

THE SOFTWARE, LOSS OF DATA, COSTS OF RE-CREATING LOST DATA, THE

COST OF ANY SUBSTITUTE EQUIPMENT OR PROGRAM, OR CLAIMS BY ANY

PARTY OTHER THAN LICENSEE.  THE OFFICERS, DIRECTORS, AND

EMPLOYEES, SHAREHOLDERS AND REPRESENTATIVES OF CYGNUS DATA

SYSTEMS ARE NOT PARTIES TO THIS AGREEMENT AND SHALL HAVE NO

OBLIGATION OR LIABILITY RELATING TO THIS AGREEMENT OR THE

SOFTWARE.

1.3.12(c).  Responsibility for Decisions.  Licensee is

responsible for decisions made and actions taken based on the

Software.  The Software is designed and intended for use by

computer professionals experienced in the uses and limitations of

computer software and it is Licensee's responsibility to

ascertain the suitability of the Software.

1.3.13.  SOLE REMEDY AND ALLOCATION OF RISK.  LICENSEE'S SOLE AND

EXCLUSIVE REMEDY IS SET FORTH IN THIS AGREEMENT.  This Agreement

defines a mutually agreed-upon allocation of risk and the License

prices and fees reflect such allocation of risk.

1.3.14.  Governing Law.  This Agreement shall be governed by the

laws of the state of California and the United States of America.

1.3.15.  Entire Agreement.  This Agreement sets forth the entire

understanding and agreement between the parties and may be

amended only in a writing signed by both parties.  No vendor,

distributor, dealer, retailer, sales person or other person is

authorized by Cygnus Data Systems to modify this Agreement or to

make any warranty, representation of promise which is different

than, or in addition to, the warranties, representations or

promises of this Agreement.

1.3.16.  Interpretation.  Any reference in this Agreement to

Software shall include any portion thereof.

ANSWER Voice Mail System 1 / Preliminary                        8

1.3.17.  Termination.  The License shall automatically terminate

if Licensee breaches this Agreement.  Upon termination of the

License, Licensee shall deliver to Cygnus Data Systems the

original and all other copies of the Software.

1.3.18.  Export Act.  Licensee hereby warrants and certifies that

no Software or any related item or product will be made available

or exported to any country in contravention of any law of the

United States, including the Export Administration Act of 1979

and regulations relating thereto.

1.3.19.  U.S. Government Restricted Rights.  The Software and

accompanying materials are provided with Restricted Rights. Use,

duplication, or disclosure by the Government is subject to

restrictions as set forth in subparagraph (b)(3)(ii) of The

Rights in Technical Data Computer Software clause at

252.227-7013.  Contractor/manufacturer is Cygnus Data Systems,

14027 NE 181st Street #B103, Woodinville, WA 98072-6846 USA.

Copyright (C) 1989-1994 Cygnus Data Systems.

All Rights Reserved.

ANSWER Voice Mail System 1 / Preliminary                        9

1.4  Contacting the Author

This documentation and software will be updated periodically.

Future software and documentation will be available to all

purchasers of this software.  Any comments and suggestions on and

to the documentation and software should be directed to:

Daniel Durbin

Cygnus Data Systems

14027 NE 181st Street #B-103

Woodinville, WA

98072-6846

(206) 481-3484 (voice)

(206) 481-9582 (bbs/fax) ZyXel U-1496e+ v.32b/v.42b

FidoNet 1:343/181

1.5  Acknowledgements

The following names are either trademarks, registered trademarks

and/or the efforts of the person and/or company named:

CompuCom                 CompuCom Corporation

DESQview                 Quarterdeck Office Systems, Inc.

DigiPhone                Cygnus Data Systems

Hayes                    Hayes Microcomputer Products Corporation

IBM-PC, PC-DOS, OS/2     International Business Machines Corp.

LANtastic Voice Adaptor  ARTISOFT, Inc.

LHarc, LHA, LZH          Haruyasa Yoshizaki

MS-DOS, Windows          Microsoft Corporation

Opus                     Wynn Wagner III

PKzip, Pkunzip, ZIP      Phil Katz, PKWare

SoundBlaster             Creative Labs, Inc.

Touch-Tone*              Bell Telephone

TurboC++                 Borland International, Inc.

Zmodem, MobyTurbo        Chuck Forsberg

ZyXEL                    ZyXEL Communications Corporation

ZOOM                     ZOOM Telephonics, Inc.

* Touch-Tone signals are also refered to as Dual Tone Multi-

Frequency (DTMF) signals.

Every effort has been made to identify and give credit for

trademarks mentioned in this documentation.  Any failure to

mention a particular trademark in the above list that may be

found in the text, or failure to give proper credit for a

particular trademark, constitutes merely an oversight and should

not be construed as intentional, or in any way a claim of rights

to the trademark.  Any mention of a particular software package

or system should not be construed as an endorsement of any kind

on the part of the authors.

ANSWER Voice Mail System 1 / Preliminary                       10

2.   Requirements

The ANSWER Voice Mail System software is designed to operate

on IBM AT class computers.  It is possible to run ANSWER on a

system with less than the following equipment, but the following

should be considered the realistic minimum:

*    An IBM AT class or compatible computer, with at least

384 Kbytes of available memory.

*    MS-DOS or PC-DOS version 3.2 or above.

*    At least 5 megabytes of available hard disk space

(mostly for message files).  ANSWER will play and

record messages to and from a floppy disk, but since

message files are large, and floppies have limited

message capacity, a hard disk is highly recommended.

The CompuCom modem, for example, samples 8 bits of

voice data at 10 Khz, which translates to 300 kbytes

per half minute.  The ZyXEL, on the other hand, samples

8 bits of voice data at 9.6 Khz, but also implements a

4 to 1 compression on the data.  So the data rate is

2400 bytes per second, or 73 kbytes per half minute.

*    A telephone line and a telephone receiver.

*    A voice mail card or modem with voice support.

*    Optionally, you may want to use a sound card such as

the SoundBlaster to record and play voice messages.

2.1  The ZyXEL U-1496 series Data/Fax/Voice modem

Requires firmware v5.00 or higher.  The latest release as of this

printing is v6.01.

2.2  The CompuCom Combo modem

Requires firmware v2.78 or higher.  The last version release was

v2.78.  I have only tested ANSWER with the CompuCom Combo

internal modem.  I have heard that ANSWER will work with some of

the external modems.  I also understand that the CompuCom Champ

internal modem can be upgraded to support voice capabilities.

Please contact me if you have or need further information.

2.3  The ZOOM Voice FaxModem

Support is provided for the VF 14.4V external and internal modems.

ANSWER Voice Mail System                3 / Introduction       11

3.   Introduction

ANSWER used with a supported voice mail card or modem with

voice support (hereby referred to as the voice mail card, or VMC)

is a voice mail and telephone answering system which features

four primary modes of operation - the Personal Answering Machine,

the Voice Mail Box, the Information Server, and the PhoneBook

Dialer.  Each mode operates independently but may access the same

messages.  Each of these modes is described briefly below, and in

detail in Chapter 5.

The Personal Answering Machine will operate just as most

common telephone answering machines, but can do even more.  It

will select from as many as sixteen different outgoing messages

and play it or them to the caller, prompt the caller to leave a

message after a tone, record an incoming message, and allow the

caller to edit the incoming message.  The outgoing message number

and the number of rings before answering can be programmed.

Incoming messages may be played back from a remote location.  A

pre-setable access code is required for this access.

The Voice Mail Box provides a system which can receive voice

messages for several persons by assigning each person a mail box

number.  A System Operator assigns mail box numbers and maintains

the mail box accounts.

The Information Server is designed to present information to

the caller in response to the caller's Touch-Tone signals.  With

this mode, the caller can selectively receive the particular

information of interest.  Callers may also leave messages in this

mode.

The PhoneBook Dialer provides an outdialing feature which

may be used to automatically dial and redial a single number

selected from a PhoneBook or sequentially dial numbers from an

ASCII list of phone numbers.  When redialing a single number from

the PhoneBook, the ANSWER will either connect data, or for voice

calls, will present an audible beep when the call goes through.

When in the list dial mode, the called party will be presented

with a menu much like in the Information Server.

In addition to telephone functions, the VMC can be used to

make your computer talk for other purposes.  Independent programs

are available which may be included in batch files to play a

"Welcome" message at boot time, or to play some sound at periodic

times of the day.  Also included are voice file format conversion

utilities.  Programming examples are also available for those who

wish to write their own programs which can control the functions

of the VMC.

ANSWER Voice Mail System                4 / Setup              12

4.   Setup

Setting up the ANSWER Voice Mail System requires the proper

installation of a VMC, and installation of the software.  Each of

these installations is described below.  Currently, the ANSWER

supports the DigiPhone VMC, the CompuCom, the ZyXEL, and ZOOM modems.

In addition, support is included for the SoundBlaster sound card.

Future versions of the software will add support for additional

hardware.  Feel free to contact me if your hardware is not

currently supported, and you would like support added.  Refer to

the modem or VMC manual for further comm port selection and

details on setting the DIP switches on the modem for the comm

port selection.

4.1  Hardware Installation

A VMC is basically a sound card with a telephone interface,

microphone, and external speaker.  At this time, the only VMC

supported by ANSWER is the DigiPhone from Cygnus Data Systems.

Two modems with voice capability are supported.  Although all

modems have telephone interfaces, most do not (if not none) have

microphones and external speaker connections.  Most high speed

modems have the ability to perform voice functions, though they

may not actually implement it.  To remain competitive, most high

speed modem manufactures are beginning to offer modems that

support Data/Fax/Voice capabilities.  As more become available,

support for them will be added to ANSWER.

A VMC has the ability to play voice, record voice, generate

Touch-Tones, and detect Touch-Tones.  The ability to detect

Touch-Tones while playing or voice allows the caller to press a

Touch-Tone while listening to a message to have ANSWER stop the

message and respond to the choice.  Most VMCs (including the

DigiPhone) can detect Touch-Tones while playing or recording

voice.  Modems may or may not be able to do this simultaneously.

The CompuCom cannot.  The ZyXEL and ZOOM modems can.

4.1.1     Modems

For Modems with voice support, you will need to identify the

communications port which the modem is attached to.  Each comm

port is identified by a number such as COM3.  However, only COM1-

COM4 have standard definitions.  Comm ports may also be

identified by their Base Address and IRQ assignments.

BASE, IRQ =    PORT #    BASE, IRQ =    PORT #

LOCAL          COM0      0x0368, 7 =    COM11

0x03F8, 4 =    COM1      0x0368, 9 =    COM12

0x02F8, 3 =    COM2      0x0278, 5 =    COM13

0x03E8, 4 =    COM3      0x03E8, 7 =    COM14

0x02E8, 3 =    COM4      0x03E8, 9 =    COM15

0x0378, 7 =    COM5      0x03E8, 5 =    COM16

0x0278, 9 =    COM6      0x02E8, 7 =    COM17

0x0368, 5 =    COM7      0x02E8, 9 =    COM18

0x0278, 7 =    COM8      0x02E8, 5 =    COM19

0x0378, 9 =    COM9      0x0268, 7 =    COM20

0x0378, 5 =    COM10     0x0268, 9 =    COM21

0x0268, 5 =    COM22

ANSWER Voice Mail System                4 / Setup              13

4.1.1.1   Addresses

The default port address used by ANSWER for the serial port

(modems) is 02E8h which is the base address for COM4.  ANSWER

allows you to select any comm port by entering the address and

the irq number for the comm port - it does not refer to the comm

port number, but simply the comm port address and the irq number.

4.1.1.2   Interrupts

The default port irq number used by ANSWER is 3 which is the

irq number for COM4.  You may select the comm port interrupt.

4.1.2     The DigiPhone

The DigiPhone requires base address selection and IRQ selection.

4.1.2.1   Interrupts

There are 5 methods of operation for voice message file

playback and recording.  The first option uses no hardware

interrupt line and is designed to work with all computer system

configurations.  However, this option will produce small audible

gaps in the voice message file during record and playback while

the file is read from the disk drive (see Options/Buffers options

for minimizing audible gap).  The other three methods use

hardware interrupts and eliminate the audible "gap" produced by

the no interrupt method.  Interrupt 0 method re-programs the 8237

timer chip for 8 Khz during message playback and record.  This

method should work with all hardware configurations, but could

possibly interfere with other terminate-and-stay-resident (TSR)

programs which also use the timer interrupt.  The next two

methods use a hardware interrupt generated by the DigiPhone

hardware.  On the AT model computer, IRQ3 may be used if no COM2

port is installed.  IRQ5 may be used if no LPT2 port is

installed.  To use either IRQ3 or IRQ5, you must set JP1 on the

card to the appropriate position.  See Figure 1 for jumper

placement.  For "No Interrupt" and "Timer Interrupt" methods,

place the jumper on "None".  For "COM2 Interrupt" and "LPT2

Interrupt" methods, place the jumper across the appropriate pins.

Method 1:  No Interrupt

Method 2:  Interrupt 0 - (IRQ0) Timer Interrupt

Method 3:  Interrupt 3 - (IRQ3) COM2 Interrupt

Method 4:  Interrupt 5 - (IRQ5) LPT2 (AT) Interrupt

Bypassing Interrupts.  As an alternative to recording

messages in short blocks and using interrupts, you may enable

Block I/O in the Options menu and not have to worry about the

interrupt setup.  Enabling Block I/O completely bypasses all

interrupt settings.  Messages are entirely recorded into memory

then saved and read completely into memory and played.  This

eliminates the small "gaps" that occur in messages when using the

interrupt mode on slow machines.  However, the length of messages

is limited by the available memory.  On a 640k machine, only

about 53 seconds of recording/playing time is available.  But

this is sufficient for most applications.

ANSWER Voice Mail System                4 / Setup              14

4.1.2.2   Addresses

The default port address used by the DigiPhone is 0278h

which is the same address used by LPT2.  You may select another

address if you have a second parallel printer port installed in

your computer by changing the DIP switch U11.  See Table 1 for

switch position information on setting U11.  An eight switch DIP

is used to select the hardware address that the DigiPhone will

occupy.  Table 1 is a summary of the possible positions of the

switch and the corresponding addresses that result.  The switch

is numbered from left to right when viewing the switch with the

card such that the edge connector is facing away from you (Hold

the card's bracket in your left hand).  With the card in this

position, switch up is on, and switch down is off.  The right two

switches are unused.  The switch will seem to be up-side-down.

The labeling on it is insignificant, and the labeling on the card

is backward (it should be labeled A8 on the right and A3 on the

left as shown in the figure).  The far right switch controls

whether the address starts with 02 or 03.  If that switch is set

(switch toward edge connector), the address will start with 03

and if clear, then 02.  The default is clear.

DIP Switch Position Setting

Switch Group        Possible Address

________________|___________________

S8               02xx - 03xx

S7 - S4          0x0x - 0xFx

S3               0xx0 or 0xx8

S2 - S1          not used

Address        Usage

0200h          (Not Used)

0201h          Game Control Adapter

0202h - 0277h  (Not Used)

0278h - 027Fh  Second Printer Port Adapter (LPT2)

0280h - 02F7h  (Not Used)

02F8h - 02FFh  Second Serial Port Adapter (COM2)

0300h - 0377h  (Not Used)

0378h - 037Fh  Printer Port Adapter Card (LPT1)

0380h - 03AFh  (Not Used)

03B0h - 03BFh  Monochrome and Printer Adapter

03C0h - 03CFh  (Not Used)

03D0h - 03DFh  Color/Graphics Adapter

03E0h - 03EFh  (Not Used)

03F0h - 03F7h  5 1/4" inch Diskette Drive Adapter Card

03F8h - 03FFh  Serial Port Adapter Card (COM1)

4.1.2.3   Volume Control

The volume control (R32 on the top of the DigiPhone card) is

set at the factory for optimum operation.  However, you may

change it to your liking by adjusting it with a small, flat

bladed screwdriver.  While looking down on the card from above

the computer, clockwise is lower volume, and counter-clockwise is

louder.

ANSWER Voice Mail System                4 / Setup              15

4.1.2.4   Jacks

There are six jacks on the DigiPhone bracket.  J1 and J2 are

the telephone jacks at the top of the bracket.  J3 - J6 are

numbered from the bottom of the bracket to the top.  The

microphone jack is the bottom jack and the speaker is the second

from the bottom (see Figure 1 on the previous page).

J1 - RJ11 modular telephone handset connector for connecting

a telephone.

J2 - RJ11 modular phone line connector for connecting the

telephone line.

J6 - External tape recorder motor drive on/off 1/16"

subminiature jack.

J5 - External tape record audio 1/8" miniature jack.

J4 - 8 Ohm speaker 1/8" miniature jack.

J3 - 600 Ohm Dynamic Microphone 1/8" miniature jack.

4.1.2.5   Inserting the VMC

With the computer power turned off, remove the five outer

screws which secure the case to the back of the computer.  Pull

the case off from the front of the computer.  You may install the

VMC in any vacant expansion slot.  Remove the blank bracket from

the back of the computer by removing one screw on the top of the

bracket.  Install the VMC in the expansion slot and use the screw

to secure the card in place.  Make sure a telephone plug will fit

through the back of the computer into the connector of the VMC.

You may have to adjust the bracket slightly to the left before

you tighten the screw to make sure the phone jack can be inserted

and removed without obstruction.  Now place the cover back on

from the front of the computer and re-install the five securing

screws.  Now you may attach the phone line and phone receiver.

The VMC hardware is now ready for use.  See Software Installation

to test the VMC hardware.  Once the Software Installation is

completed, you are ready to run ANSWER.  Please read the Software

Installation section of this chapter.

ANSWER Voice Mail System                4 / Setup              16

����������������������������������������������������������������������������͸

�  Phone       Messages     Activate      Control       Help       Options   �

� Direct control of the phone line                                           �

����������������������������������������������������������������������������;

���     ��   ��   ������   ��   ��   ������   �������

��   ��   ���  ��   ��       ��   ��   ��       ��   ��

�������   �� � ��   ������   ��   ��   ����     �� ���

��   ��   ��  ���       ��   �� � ��   ��       ��   ��

��   ��   ��   ��   ������    �� ��    ������   ��   ��

Copyright (C) 1989-94 by Cygnus Data Systems

Version 3.7f �  Release 05/15/94

�������������������͸  ������������������������������Ŀ �������������������͸

�BIN Dir:   29927424�  �                              � �This software is   �

�ICM Dir:   29927424�  �                              � �Registered to one  �

�OGM Dir:   29927424�  �                              � �owner and may not  �

�PGM Dir:   29927424�  �                              � �be distributed.    �

�TMP Dir:    2088960�  �                              � �                   �

�Voice Card Working �  �                              � �                   �

�������������������;  �������������������������������� �������������������;

���������������������������������������������������������������������������Ŀ

�  Freq: 10000 System: AT 01FC         05/15/94 19:47:13  Free Mem: 362304  �

�  EXIT: ESC   �   PHONE: On  Hook  �   MESSAGE: None     �  Calls:    0    �

�����������������������������������������������������������������������������

4.2  Software Installation

Detailed installation instructions are given below for the

software.  For a quick start installation, and current release

notes, please see the README! file in the distribution archive.

4.2.1     Installing The Software

You may simply copy all the files from the distribution

diskette to a directory of your choosing.  The message files

required for operation are archived and should be unarchived

using the unarchiver program provided.  Also, please refer to the

README! file on the distribution diskette for additional

information and software changes after the printing of this

document.

4.2.2.1   First Run

The first time you run ANSWER, it will create a data file

called OPTIONS.DAT.  You must visit the Options menu and set your

desired options.  The default values are selected to work on most

computer systems.  However, you should review them before doing

anything else with the software.  For further information on the

options menu, see the Options section in the Operations chapter.

A brief description is provided for setting up each of the

four modes of operation follows.  The steps described are the

minimum required to operate each mode.  There are options

available not mentioned in the discussion which may be set to

alter the operation.  See chapter 5. Operations for a detailed

description of each of the available options.

ANSWER Voice Mail System                4 / Setup              17

4.2.2.2   Answering Machine

To setup ANSWER as an answering machine, you must specify

which outgoing message will be played, which directory outgoing

messages are stored in, what the time limit is on incoming

messages, and where to put incoming messages.  Follow the

procedure below to accomplish this.

Select Options/Directories/Outgoing Messages.  Enter the

directory name where outgoing messages are stored.  If the

directory does not yet exist, ANSWER will ask you if it should

create the directory.  The default directory is the current

working directory, but a good alternative would be C:\OGM.

Select Options/Directories/Incoming Messages.  Enter the

directory name where incoming messages are stored.  If the

directory does not yet exist, ANSWER will ask you if it should

create the directory.  The default directory is the current

working directory, but a good alternative would be C:\ICM.

Select Messages/Outgoing/Edit.  You may type in up to four

lines of text which are not used by ANSWER, but may be used by

you as notes on the current outgoing message.  You may use this

space to type out your message before you record it.  When you

are ready to record, press F9 and begin speaking.  ANSWER will

wait for you to begin speaking, start recording, pause when no

sound is heard, and stop recording after a 3 second pause.

Outgoing messages are limited to 90 seconds by default.

Select Control/Answering Machine/OGM Number and activate the

outgoing message which will be played for each incoming call.

Only messages which you have already recorded may become active.

You may have up to sixteen outgoing messages available to select

from and any number of them may be active at any time.  To make

an outgoing message active, use the UP/DOWN ARROW keys to

highlight an outgoing message that you have already recorded, and

press ENTER.  Then use the RIGHT/LEFT ARROW keys and the 'Y' and

'N' keys to choose the days on which the message will be active.

You may also specify the times during the day that the message

will be active.  The hours are on the 24 hour clock basis, where

hours after 12:00 noon are greater than 12 (for example, 3:00pm

would be 15).  Enter the beginning time and end time of the

active period.

Select Control/Answering Machine/Rings and enter the number

of rings to allow before answering the phone.  ANSWER will answer

the phone after the number of rings specified here have occurred.

Select Activate/Answering Machine.  ANSWER will now answer

the telephone after the specified number of rings, deliver the

specified outgoing message (OGM#), record an incoming message

(ICM#) of up to the specified maximum number of seconds, and hang

up the phone.  The number of incoming calls received will be

updated and displayed in the status box in the lower right hand

corner of the screen.  After 10 seconds of keyboard inactivity,

the screen will go blank and a message will begin flashing in the

middle of the screen indicating that ANSWER is waiting for calls.

If any incoming calls have been received, the number of calls is

ANSWER Voice Mail System                4 / Setup              18

displayed below the waiting message.  The screen will be restored

when any key is pressed.

ANSWER is now ready to answer calls.  To test the operation

of the answering machine, you may call it from another phone

number, or you may simulate the receipt of an incoming call by

pressing ALT-C.  The answering machine will continue as if it had

received a call, including picking up the telephone.

4.2.2.3   Voice Mail Box

To setup ANSWER as a voice mail box system, you must specify

the number of digits to be used for mail box extensions and

passwords.  You must also establish the mail box database by

entering each mail box holder's name, box number, password, etc.

As in the answering machine mode you must specify which directory

outgoing messages are stored in, what the time limit is on

incoming messages, and where to put incoming messages.  Follow

the procedure below to accomplish this.

Select Options/Hardware/Limits.  Enter the number of maximum

seconds allowed that will be allowed for recording incoming,

outgoing, and Voice Menuing messages.

Select Options/Directories/Outgoing Messages.  Enter the

directory name where outgoing messages are stored.  If the

directory does not yet exist, ANSWER will ask you if it should

create the directory.  The default directory is the current

working directory, but a good alternative would be C:\OGM.

Select Options/Directories/Incoming Messages.  Enter the

directory name where incoming messages are stored.  If the

directory does not yet exist, ANSWER will ask you if it should

create the directory.  The default directory is the current

working directory, but a good alternative would be C:\ICM.

Select Control/Voice Mail/User-Extension Edit.  When this is

selected for the first time, the user database is established

using the extension and password digit length.  If these lengths

are ever changed, the user database must be re-established.

Enter each user's name, extension, password, and the user's

introduction message filename if any.

Select Activate/Voice Mail.  The ANSWER is now ready to be

operated in the Voice Mail Box mode.  ANSWER will now answer the

telephone after the specified number of rings, and prompt the

caller for a mail box extension.  If the extension is available,

the introduction message (if available) will be played.  The

caller may then leave a message.

ANSWER Voice Mail System                4 / Setup              19

4.2.2.4   Information Server

Control of the Information Server is done by editing an

ASCII file called TREE.CFG.  An example file is provided with the

software.  ANSWER begins by playing the message file called

MAINMENU.BIN which must be the first file listed in the TREE.CFG

file.  Under this entry, you may list other entries which will be

branched to when the caller presses a Touch-Tone button.  To list

these entries, enter a TAB character, the number corresponding to

the Touch-Tone, a comma, and the name of the message file to

branch to.  You may do this for up to 10 messages.  The number 0

is listed as 10 and defaults to MAINMENU.BIN.  The '#' button is

number 11 and is used to backup one menu.  The '*' button is

number 12 and is used to abort the voice mail session.  When this

button is pressed, BYE.BIN will be played and the phone will be

hung up.  The Information Server ends with a file called BYE.BIN

which must be the last file listed in TREE.CFG.  Under this

entry, you should put the command EXIT.  You may use any ASCII

text editor to edit the TREE.CFG file.  You may also edit this

file by selecting Control/Information Server.

In addition to branching to other messages, ANSWER may also

be instructed to RECORD, PLAYBACK, and EXIT.  The RECORD command

can be used to receive an incoming message from the caller.  The

incoming message will be named just as in the Answering Machine,

and will be stored in the ICM directory, and is limited to the

specified number of maximum seconds.  The PLAYBACK command is

used for demonstration purposes.  It will allow the caller to

record a message and here it played back.  This message is called

TEST.BIN and is placed in the temporary directory, but is deleted

after being played back to the caller.

4.2.2.5   PhoneBook Dialer

This mode allows you to use ANSWER to make outgoing calls.

You should enter your most often dialed numbers into the phone

book.  When you want to call a number, simply use the ARROW keys

to highlight your desired number and press return.  ANSWER will

take the phone off hook, look for a dialtone, dial the number,

look for ringing or busy, and respond appropriately.  If the call

fails, it will retry until a key is hit.  If the phone number

TYPE is DATA and if a CONNECT is received, ANSWER will exit with

an errorlevel equal to the numeric result code of the data

connection.  If VOICE is selected as the phone number TYPE, and

If the call goes through and is answered, the Information Server

is invoked.

You may also dial from a list of numbers.  The list must

contain a new telephone number as the first thing on each line.

You may enter comments after the number by preceding your

comments with a tab and a semicolon.  The numbers are dialed

exactly in the same way as the PhoneBook dialer.

ANSWER Voice Mail System           5 / Operation               20

5.   Operation

This chapter describes in detail each of the menu and submenu

options.  There are six main menu options: Phone, Messages,

Activate, Control, Help, and Options.  Each is listed below with

each of their submenu functions fully detailed.  Use the ARROW

keys, the first letter of the menu or submenu command, or the

mouse to select the choices.  Use the ESCAPE key to return to the

previous menu.  Pressing ESCAPE from the Main Menu (no pull down

window displayed) exits ANSWER.  Also, at any time during message

recording or playing, a keyboard press will stop the process.  If

the key is a valid Touch-Tone button, it will be interpreted as

such.  In addition, the keyboard numbers may be pressed in

response to any Touch-Tone prompt.

5.1  Phone

This menu provides direct manual control over the phone

interface.

5.1.1     On Hook

Manually puts the telephone line on hook - hangs up phone.  The

status of the phone hook is displayed on the status line.

5.1.2     Off Hook

Manually takes the telephone off hook - picks up phone.  This

will terminate any data connection by lowering DTR, waiting one

second, and raising DTR.

5.1.3     Dial

Simply dials a number (up to 16 digits) with no conditions.

Commands the VMC to dial a number and waits for an OK response,

then returns control to the menu.  If a CONNECT code is received,

then the Terminal mode is entered.  If a keyboard key is pressed,

or a timeout (35 seconds) occurs waiting for a response, the

dialing is aborted.

5.1.4     Flash

Places the phone on hook for 1 second, then off hook.

5.1.5     Terminal Mode

Data Communications Terminal.  Provides a raw terminal mode for

use in calling out via data communications.  Commands available

in the terminal mode are ALT-X to exit, ALT-D for dialing menu,

ALT-C to clear the screen, ALT-H to hang-up. ALT-F4 to shell to

DOS, PAGE-UP to send a file by calling PAGEUP.BAT, and PAGE-DN to

receive a file by calling PAGEDN.BAT.  For diagnostics, ALT-N

performs a "receive Touch-Tone" test.  In this mode, any Touch-

Tone received will be displayed.  ALT-T performs a "tone" test.

In this mode, press a number from 0 to 7 to hear that tone

played.  The communications parameters are set to 19200,N,8,1.

The Online/Offline indicator on the status line continually

reflects the status of Data Carrier Detect (DCD).

ANSWER Voice Mail System           5 / Operation               21

5.2  Messages

Because most modems do not have external speakers or microphones,

the telephone receiver is used to play and record messages.  In

order to play and record messages with the telephone receiver, it

must be taken off hook and in order to silence the dialtone, a

single digit is dialed every 15 seconds by the modem.  For some

installations, an external box is available will simulate the

phone line power so that dialing is not necessary.  Also, if the

modem or VMC is called from another telephone number, message

recording and playing may proceed without dialing.  If the modem

is already off hook before going into play or record mode, it

will not dial every 15 seconds.  If the modem is on hook, it will

dial digit 4 every 15 seconds.

5.2.1     Play

Manually plays any file as specified.  The default extension for

voice files is *.bin.

5.2.2     Record

Manually records any file as specified.  If a sound card is

detected, it will be used.  If the Null Phone Number is not

blank, it will be dialed instead of the single digit 4.  No file

is saved if no voice is recorded or an error occurs.  A

"Recording" Status is displayed on the status line.  If a Touch-

Tone button is pressed, a keyboard key is pressed, or a 3

(programmable) second silence period is detected, the recording

is stopped.

5.2.3     InComing

Allows you to review incoming messages, play them, and delete

them as appropriate.  All incoming messages begin with "ICM" and

end with ".BIN" with the message number in between, for example,

"ICM1.BIN".  When incoming calls are recorded, the first

available ICM number is used.  For example, if ICM1.BIN and

ICM3.BIN exist, but ICM2.BIN was previously deleted, the next

recorded incoming message will be ICM2.BIN.  The Call Log

(ANSWER.LOG) keeps record of all incoming messages.

5.2.3.1   Play

Selectively plays an incoming message from the ICM directory.  If

the VMC or modem is already off hook, the message will just be

played.  If not, it is taken off hook, a single digit number is

dialed, the message is played, and the modem is placed on hook.

However, if a sound card is detected, the message is played

through it instead.

5.2.3.2   Call Log

Displays a record of incoming call activity.  Use PAGE-UP, PAGE-

DN, HOME, and END to move through the log file (an ASCII text

file named ANSWER.LOG).  The format is as follows:

ANSWER Voice Mail System           5 / Operation               22

From the Opus doc file OREF_LOG.LZH:

* Each line consists of these parts:

SYMBOL.....    !  - an important entry, usually an error

*  - network

+  - a regular entry

:  - a non-important message

#  - tracer statements

_  - <space> video message only, not logged

DAY.............. one or two digits

MONTH............ three-character month name

TIME............. 24-hr clock in the form hh:mm:ss

PROGRAM.......... as many as 8 characters/digits, no spaces

MESSAGE.......... as many as 80 characters/digits/etc

END-OF-LINE...... A cr/lf for MsDOS

That comes to about 107 characters as the maximum line length.

5.2.3.3   Times/Dates

Displays the date and time stamp of incoming message files.

5.2.3.4   Delete

Removes an incoming message from the hard disk.  The number of

incoming calls is displayed in the lower right hand corner of the

screen and will be updated.  You should check incoming messages

often and delete them as soon as you can since they can very

quickly fill up the hard disk.  Keep an eye on the "Free Disk

Space" box.  If the free space gets too low, the box will turn

red and an audible beep will be played periodically.

5.2.3.5   Record

Locally records an incoming message.  The selection box

automatically selects the next unrecorded ICM number and

increments the counter if the recording was successful and it was

a new message.

5.2.4     OutGoing

Outgoing Message Management.  Allows you to play, record, and

edit outgoing messages.

5.2.4.1   Record

Record an outgoing message.  Outgoing messages must begin with

"OGM" and end with ".BIN" and have the appropriate number in

between.  For example "OGM1.BIN".  The selection box

automatically selects the next unrecorded OGM number, and

increments the counter if the recording was successful and it was

a new message.

5.2.4.2   Select

Selects an outgoing message for manipulation.  (This function

performs nothing at this time).

ANSWER Voice Mail System           5 / Operation               23

5.2.4.3   Edit

Edits an outgoing message.  This option allows you to enter up to

four lines of text related to the selected outgoing message.

This text may be the exact wording you want in the message.  This

allows you to read the text while recording the message.  After

entering the text, simply press the F9 key to begin recording.

To hear the message played back, press the F10 key.  You may

advance and rewind through the message by using the ARROW keys.

As you do this, the file counter will be updated which indicates

the file position in increments of the Card Frequency divided by

ten (Freq/10).  You may begin recording over the current position

in the outgoing message allowing you to redo parts of the

message.

5.2.4.4   Delete

Deletes any outgoing message.  This also removes any text, any

programming that was entered for the selected outgoing message,

and updates the ogm counter.

5.2.4.5   Play

Plays any outgoing message.

5.2.5     Examine

Allows you to view recorded messages in a graphic "OSCILLOSCOPE"

form and in a "FREQUENCY DOMAIN" mode.  Also provides a

diagnostic menu for displaying the comm port, or VMC status.

5.2.5.1   Graphic Editor

Graphically displays any message file in a "Time Domain" or

"Oscilloscope" form.  This form is time (across) verses amplitude

(up).  You may use the ARROW keys (RIGHT, LEFT, UP, and DOWN) to

move the graphic cursor 100 steps, the PAGE-UP and PAGE-DN keys

to scroll an entire page, the HOME and END keys to move from one

side of the screen to the other, and the CTRL-HOME and CTRL-END

keys to move from the beginning of the file to the end of the

file.  You may mark a block of the file by positioning the

graphic cursor at the beginning of the block an pressing '1'.

Mark the end of the text by positioning the graphic cursor and

pressing '2'.  Write the block to a new file (called OUT.BIN) and

pressing "shift-W" (upper case w).

5.2.5.2   Spectrum Analyzer

Displays any message file in a "Frequency Domain" form, that is,

frequency (across) by amplitude (up).  Each channel represents

128 hertz.  The display is updated as quickly as possible, but

does not display in real time.

ANSWER Voice Mail System           5 / Operation               24

5.2.5.3   Diagnostics

Comm Port Status or VMC Status.  Displays the state of the serial

port by displaying abbreviations for the flags in the UART's

Modem Status Register and Line Status Register, and identifies

the type of UART.  The definitions of the abbreviations used are

listed below.  Press Escape to exit diagnostics.

Modem Status Register (MSR)   Line Status Register (LSR)

---------------------------   --------------------------

DCTS Delta Clear-To-Send      RXRD Received Data Ready

DDSR Delta Data-Set-Ready     OVRN Overrun Error

DRNG Delta Ring Detect        PARY Parity Error

DDCD Delta Data-Carrier-DetectFRAM Framing Error

CTS  Clear-To-Send            RBRK Received Break

DSR  Data-Set-Ready           TXRD Transmit Ready

RNG  Ring Detect              TXHE Transmit Hold Reg Empty

DCD  Data-Carrier-Detect

For VMCs, the status is shown as below:

63        0        Status Register     Single Bit Register

Not Clock          State of the Clock

No DTMF            Status of the DTMF (Touch-Tone) Receiver

Silence            Status of the Silence Detector

No Ring            Status of the Ring Detector

On Hook            Status of the Hook

0                  Amplitude of the audio

Freq: 8000 hz      Frequency of the Clock

5.3  Activate

The activation menu is used to invoke one of the four modes of

operation.  Each of these modes can be invoked directly from the

command line.  For more information on command line parameters,

see chapter 8.

5.3.1     Answering Machine

This is the Personal Answering Machine.  In this mode, a

specified outgoing message is played for every call, after which

the caller will be allowed to leave a message limited by a

maximum number of seconds.  If the caller hangs up, or if 3

seconds of silence occurs, or the FFT routines detect a dialtone,

then the session will be terminated.  After the outgoing message

or messages are played, a pause will occur to check for a special

function requested by pressing a Touch-Tone button.  After 4

seconds, the tone will be played to indicate to the caller to

begin recording the incoming message.

5.3.1.1   Wait for Calls

While waiting for calls, the screen is blanked after 10 seconds

of inactivity.  If a keyboard key is pressed or a ring is

received, the screen is restored.  Once the specified number of

ANSWER Voice Mail System           5 / Operation               25

rings are received, the detected call is written to the log file.

You may simulate reception of a call by pressing ALT-C.  Pressing

ESCAPE exits the Answer Mode.  In the automatic mode, this

section is skipped, but a call is still logged.

5.3.1.2   Answering the Phone

The phone is taken off hook.  If the Control/Answer/Answer Mode

is set to No, then the call proceeds with OGM delivery.

If the Control/Answer/Answer Mode is set to Data, then a Data

connection is attempted as follows.  If the Control/Answer/Answer

Mode is also set to Prompt, then the "PROMPT.BIN" message is

played, otherwise it is not.  If a Touch-Tone is received within

the timeout, then the call proceeds with OGM delivery assuming a

human caller.  If a Touch-Tone is not received, then "ATA" is

sent to the modem to attempt a Data connect.  If a connect is

received, then ANSWER drops with an errorlevel equal to the

numeric result code of the connection.  If no connect is made,

then the phone is placed on hook and ANSWER returns to waiting

for calls.

If the Control/Answer/Answer Mode is set to Fax, then a Fax

connection is attempted as follows.  If the Control/Answer/Answer

Mode is also set to Prompt, then the "PROMPT.BIN" message is

played, otherwise it is not.  If a Touch-Tone is received within

the timeout, then the call proceeds with OGM delivery assuming a

human caller.  If a Touch-Tone is not received, then ANSWER drops

with a Fax Code errorlevel of 33.

5.3.1.3   Outgoing Messages Delivery

Each outgoing message which is active for the time of the call is

played.  For those modems or voice cards that detect Touch-Tone

signals during message delivery, the reception of a Touch-Tone

aborts the message delivery.  Please note that if no OGMs are

active, none are played and logged as having been played.

5.3.1.4   Listen for Touch-Tone

Once all active OGMs are played, ANSWER waits a timeout period

for a Touch-Tone to indicate one of the below special functions.

5.3.1.5   Page Operator

If the caller presses '0' after the OGM delivery but before the

ICM prompt Tone 3, the System Operator is paged.  "PAGE.BIN" will

be played, then a tone will be audible over the PC's speaker for

a period of 30 seconds.  If the System Operator does not respond,

then the call proceeds with ICM recording.

5.3.1.6   Fax Code Errorlevel Exit

If the caller presses '*' after the OGM delivery but before the

ICM prompt Tone 3, ANSWER exits with the Fax Code errorlevel of

33.  This is used to have the modem/fax card receive a fax.

ANSWER Voice Mail System           5 / Operation               26

5.3.1.7   Remote Mode

If the caller presses '#' after the OGM delivery but before the

ICM prompt Tone 3, the caller is prompted for the Remote Mode

access code.  This allows you to call ANSWER from another number

and have it play all active incoming messages.  You will hear a

Tone 3, then you must then enter the access code ('5' is the

default).  If the access code matches, remote mode will be

entered indicated by Tone 11.  The "REMOTE.BIN" and "RMOPEN.BIN"

messages are played.  Then a Tone 3 is played and you may enter a

Touch-Tone signal.  At this point you have several options.

Press '1' to hear each incoming message.  ANSWER will beep a

number of times equal to the number of active incoming messages

and then play each one in succession.  Press '5' to execute a

batch file called REMOTE.BAT.  Press '#' to delete all incoming

messages.  Press '*' to exit Remote Mode.  If no Touch-Tone is

received after a timeout, the "TIMEOUT.BIN" message is played.

After several timeouts the call is terminated.  Once the Remote

Mode is completed, the "EDITDONE.BIN" message is played and

ANSWER returns to waiting for calls.

5.3.1.8   Incoming Message Recording

Tone 0 is played to indicate the ICM is being recorded.  Then

"ICMEDIT.BIN" is played.  If no Touch-Tone is received,

"TIMEOUT.BIN" is played.  After several timeouts, the editing is

completed and the message is saved.  If '0' is received,

"SAVED.BIN" is played and the editing is completed.  If '1' is

received, Tone 0 is sounded and the message is re-recorded.  If

'2' is received, the ICM is played back.  If '3' is received,

Tone 0 is played and the ICM is appended.  If '*' is received,

the ICM is erased and editing is competed.  If the ESCAPE key is

pressed, the session is canceled.  Once the editing is completed,

"EDITDONE.BIN" is played and the function returns.

5.3.1.9   Session Completion

The "BYE.BIN" message is played.  The phone is placed on hook and

the system returns to waiting for calls if not in automatic mode.

5.3.2     Voice Mail Box

This is the Voice Mail Box system.  This allows several people to

receive voice mail.  Each person has a mail box number which the

caller uses to direct voice mail to the person.  To use this

system, the callers must know the extension number they wish to

leave messages to.  Also, each person receiving messages has to

have been assigned a voice mail box by the System Operator who

maintains the voice mail box accounts.

5.3.2.1   Wait for Calls

See Answering Machine/Wait for Calls.

5.3.2.2   Answering the Phone

See Answering Machine/Wait for Calls.  If a human caller is

detected, the caller will be prompted with "VMOPEN.BIN" to enter

ANSWER Voice Mail System           5 / Operation               27

the voice mail box number (extension), and Tone 3 will be sounded

to prompt for an extension.  If the caller enters an '*', the

System Operator will be paged.  If the extension is not

recognized, "VMRETRY.BIN" will be played to prompt the caller to

reenter the voice mail box number.  The call proceeds with either

the Mail Box Number prompt or the Edit Extension prompt.  Once

the session is complete, if not in automatic mode ANSWER returns

to waiting for calls.

5.3.2.3   Mail Box Number

Once the caller enters a valid mail box number, if the box holder

has an introduction message, it will be played.  Then Tone 3 will

be sounded to indicate the ICM is being recorded.  The caller

will then be presented with the ICM edit menu.  If the recording

is successful, it will be saved to the ICM directory with the

voice mail box naming convention.

5.3.2.4   Edit Extension

If the Edit Extension code is entered, the caller is prompted for

the mail box number to edit and a password for access.  Then the

caller is presented with "VMEDIT.BIN" editing menu.  This allows

mail box holders to retrieve their messages.  The selections are

'1' to delete the message, '2' to play the message, '3' to play

next message, '4' to play previous message, or '#' to end

editing.

5.3.3     Information Server

This is the Menu'ed Answering Mode, used for information services

and is controlled by the TREE.CFG file.  This mode can be used by

the caller to selectively receive information on a variety of

different subjects.  The caller may also leave messages in this

mode.  See Control/Information Server for information on the

TREE.CFG configuration file.  All menu message files listed in

the TREE.CFG file must reside in the OGM directory.

The same Wait for Calls and answering sequence as in the Answer

Mode takes place here.  If a voice call is received, the screen

displays the Information Server Menu.  The caller is presented

with the "MAINMENU.BIN" message.  Then the Tone 3 is sounded

prompting the caller for a Touch-Tone.  An '*' branches to the

"BYE.BIN" branch which terminates the call.  A '#' branches to

the previous menu, or to "BYE.BIN" from the "MAINMENU.BIN".  A

'0' returns to the "MAINMENU.BIN".  The '1' through '9' keys are

defined in the TREE.CFG file (see Control/Information Server).

After a timeout, "TIMEOUT.BIN" plays prompting the caller for a

Touch-Tone.  After several timeouts, the call is terminated.  If

a branch has no definition, "INVALID.BIN" is played.  The entered

key is repeated back to the caller ("NUMS.BIN").  Then the branch

or key word function is called.

5.3.4     PhoneBook Dialer

This is the OutDialer mode.  ANSWER can be use as a redialer to

call a number entered is a PhoneBook, or from a list of numbers,

or a manually entered number.

ANSWER Voice Mail System           5 / Operation               28

5.3.4.1   PhoneBook

Maintains a list of numbers for outdialing.  See

Control/PhoneBook for further information.

5.3.4.2   FileList

Selects numbers from an ASCII text for outdialing.  Each number

is read from the file and a call is attempted (see Outgoing Calls

below).  If any call is unsuccessful, the dialing is aborted.  If

the call is successful, the Information Server is invoked.

Note:  This function could use some improvement.  Namely, it

shouldn't abort after an unsuccessful call, but should log

success or failure to a file and continue.  But since automated

dialers are controversial, no further work is planned for this

function.

5.3.4.3   Manual

Manually dials a phone number and tries to connect.

5.3.4.3.1  Outgoing Calls

The phone is taken off hook and checked for a dial tone.  If

none, then it aborts.  Then it dials the number.  If a keyboard

key is pressed it aborts.  If busy, it retries.  If it times out,

it retries.  If it connects data, it returns success.  If it gets

a "VOICE" result it continues.  If it "RINGS", it continues.  If

it detects a Touch-Tone, it aborts.  If the call continues, the

success is logged with date and time.

5.4  Control

This menu allows you to set modes of operation.

5.4.1     Answering Machine

Select the outgoing message to be played in the Answering Machine

mode.  Also, set the maximum number of seconds for incoming

calls, set the number of rings to wait before answering, and set

the remote access code.  Select the code length and code for

remote access.

5.4.1.1   OGM Number

You may have up to sixteen different outgoing messages available

to play to callers.  These may be active depending on the day of

the week and the time of the day.  Any of these may be active at

any one time.  An outgoing message must be already recorded to

become active.  Use the UP/DOWN ARROW keys and the ENTER key to

activate/deactivate outgoing messages.  Use the RIGHT/LEFT ARROW

keys and the 'Y' and 'N' keys to select the active days.  Enter

the beginning and end active times using the numeric keys.  Times

are in the 24 hour clock format so that hours after 12:00pm noon

are greater than 12.  For example, 3:00pm would be 15.  Midnight

is 0.  It is possible to have several outgoing messages active at

any one time.  If the hours overlap for outgoing messages, they

are each played sequentially.

ANSWER Voice Mail System           5 / Operation               29

#   Active   M  T  W  Th F Sa Su   Hours  Available

1    Yes      Y  Y  Y  Y  Y  Y  Y   0  23  Yes

2    No       N  N  N  N  N  N  N   0   0  No

3    No       N  N  N  N  N  N  N   0   0  No

4    No       N  N  N  N  N  N  N   0   0  No

5    No       N  N  N  N  N  N  N   0   0  No

6    No       N  N  N  N  N  N  N   0   0  No

7    No       N  N  N  N  N  N  N   0   0  No

8    No       N  N  N  N  N  N  N   0   0  No

9    No       N  N  N  N  N  N  N   0   0  No

10   No       N  N  N  N  N  N  N   0   0  No

11   No       N  N  N  N  N  N  N   0   0  No

12   No       N  N  N  N  N  N  N   0   0  No

13   No       N  N  N  N  N  N  N   0   0  No

14   No       N  N  N  N  N  N  N   0   0  No

15   No       N  N  N  N  N  N  N   0   0  No

16   No       N  N  N  N  N  N  N   0   0  No

[ESC]:exit  [RET]:toggle active  [ARROWS]:move  [Y/N]:days

5.4.1.2   Rings

Enter the number of rings (1 to 9) to wait before answering the

phone.

5.4.1.3   Remote Code

Enter the secret access code length (1 to 8) and the code itself.

Use this access code to call ANSWER while you're away.  With this

code, you may gain access to your incoming calls and delete them

after listening to them.

5.4.1.4   Answer Mode

a) Detect Data/Fax calls or Not (choices: D/F/N)

b) Play PROMPT.BIN or not (choices Y/N)

When set to detect Data or Fax calls, ANSWER will wait 5 seconds

after answering the phone for a Touch-Tone.  But when set to

Prompt, ANSWER will first play "PROMPT.BIN" before waiting for

the Touch-Tone.  If received, voice interaction will continue.

Else, ATA is sent to the modem.  If a connect is received, ANSWER

will exit with an errorlevel equal to the numeric result code of

the connection or 33 if CONNECT FAX.  See the modem User's Manual

for the numeric result codes.

5.4.2     Voice Mail Box

This feature allows several people receive private messages by

assigning each person a voice mail box number and password.  The

Systems Operator sets up the voice mail accounts through this

menu.

5.4.2.1   Extension Digits

Enter the number of digits each voice mail box will require to

access it.  The minimum is 2 digits and the maximum is 9 digits.

ANSWER Voice Mail System           5 / Operation               30

Please note that this establishes the maximum number of voice

mail boxes are available.  For 2 digit extensions, 99 boxes are

possible.  However, the maximum overall limit is 256 regardless

of the number of digits used for extensions.  If this is changed,

the user database will have to be re-established.

5.4.2.2   Password Digits

Enter the number of digits for the voice mail box owner's

password required to access it.  The minimum is 2 digits and the

maximum is 9 digits.  If this is changed, the user database will

have to be re-established.

5.4.2.3   Edit Extension No.

Enter the access code for voice mail box owners to access their

voice mail boxes.  This extension number is entered at the "Enter

the extension you wish to reach" (VMOPEN.BIN) prompt by each of

the mail box holders to access the play messages menu.

5.4.2.4   Incoming Msgs

Examines InComing messages.

5.4.2.5   User/Extension Edit

The System Operator assigns the voice mail boxes from this menu.

Each mail box owner receives a mail box number, password, and may

record an introduction message.  The user database also contains

the user's name and a phone number where the user can be reached.

If the user database does not yet exist when this function is

selected, the database will be created using the digit lengths

set above for extension and password.  If those lengths are

changed after establishing the database, the database will have

to be re-established.  Follow these steps to re-establish the

database:

a)   Delete the VMAIL.SYS and VMAIL.DEF files in the PGM

directory.

b)   Set the Extension Digits and the Password Digits.

5.4.3     Information Server

This menu offers three different methods of editing the TREE.CFG

file, an ASCII text file which is used to control the operation

of the Information Server by the use of keywords which are

described under Edit Menu below.  You may used any ASCII text

editor to edit this file.  An example configuration file is

provided on the software distribution disk and contains an

example of how to set up the Information Server menu structure.

Several commands are available and each is used in the example.

5.4.3.1   Edit Menu

This is a menu driven configuration file editor.  When selected,

it will prompt for the name of the configuration file to edit.

Once a filename to edit is chosen, the file is read and a menu

box is presented as shown below.

ANSWER Voice Mail System           5 / Operation               31

Editing tree for

Key    Choice        Previous

TM [mainmenu.bin] [     bye.bin]

1 [ invalid.bin] [            ]

2 [ invalid.bin] [            ]

3 [ invalid.bin] [            ]

4 [ invalid.bin] [            ]

5 [ invalid.bin] [            ]

6 [ invalid.bin] [            ]

7 [ invalid.bin] [            ]

8 [ invalid.bin] [            ]

9 [ invalid.bin] [            ]

0 [     bye.bin] [            ]

* [     bye.bin] [            ]

# [mainmenu.bin] [            ]

[ESC]  [RET]:edit  [ARROWS] [DEL]

The menu box lists branch menu definitions for each Touch-Tone

selection.  Use the ESCAPE key to either back up one branch menu

if any, or terminate the editing session and save the

configuration file.  Use the RETURN key to edit the branch menu

definition.  If no branch is yet defined, you will be prompted

for a branch menu filename.  The box is then redrawn for editing

the branch menu definition.  Use the ARROW keys to move between

the branch menu selections.  Use the DELETE key to remove a

branch definition.

5.4.3.1.1 Top Menu (TM)

The first line states "Editing tree for" and refers to the branch

listed to the right of TM.  Notice that "MAINMENU.BIN" is always

the first menu.  The second line states "Key Choice Previous".

5.4.3.1.2 Key

The "Key" column refers to the Touch-Tone signal entered by the

caller.  Valid keys are '0' through '9', '*', and '#'.  Notice

that "INVALID.BIN" appears after each choice which does not yet

have a branch menu defined.  Also, "BYE.BIN" always appears after

the '*'.  This choice is offered to the caller at each branch

menu and can be selected by the caller to "Abort" or terminate

the call from any branch menu.  Additionally, notice that the

previous branch menu always appears after the '#'.  This choice

is offered to the caller at each branch menu can be selected by

the caller to "Back up one menu" and return to the previous menu.

5.4.3.1.3 Choice

The "Choice" column refers to the branch menu which is called

when the Key is pressed.  "INVALID.BIN" is automatically entered

for any key which does not have a branch menu definition.

ANSWER Voice Mail System           5 / Operation               32

5.4.3.1.4 Previous

The "Previous" column refers to the branch menu from which the

current menu is called.  Notice that "BYE.BIN" is always at the

top of the Previous column.  When the caller presses '#', control

is returned to the branch menu at the bottom of this column.

5.4.3.1.5 Key Words

The Information Server menu begins with MAINMENU.BIN.  All

selections from the MAINMENU must define branch menus.  When

editing a branch menu, pressing RETURN on a selection brings up

the keyword box.  Use the ARROW keys to select the desired

keyword, or select "Edit Branch" to branch to a submenu.

5.4.3.1.5.1    Edit Branch

Creates a new branch menu or edits an existing branch menu

definition.  From the MAINMENU, branch menu filenames are

possible.  From branch menus, additional branch menu filenames

and key words are possible.  You must first record the branch

menu message and place it in the OGM directory.

5.4.3.1.5.2    Record

When the caller selects a key with this keyword as its choice,

ANSWER records an incoming message and saves it to the ICM

directory with the normal ICM naming convention and the ICM

number counter is incremented.

5.4.3.1.5.3    Playback

When the caller selects a key with this keyword as its choice,

ANSWER records a temporary message in the TEMP directory, plays

it back to the caller, then deletes it.

5.4.3.1.5.4    Yell

When the caller selects a key with this keyword as its choice,

ANSWER yells for (pages) the System Operator.  If there is no

answer within a timeout, the caller is informed that the System

Operator is unavailable.

5.4.3.1.5.5    Exit

When the caller selects a key with this keyword as its choice,

ANSWER terminates the call (branches to "BYE.BIN").

5.4.3.1.5.6    Remote

When the caller selects a key with this keyword as its choice,

ANSWER enters the remote mode (see Activate/Answer/Remote Mode).

5.4.3.1.5.7    GetDTMF

When the caller selects a key with this keyword as its choice,

ANSWER prompts the caller for a DTMF (Touch-Tone) signal.

ANSWER Voice Mail System           5 / Operation               33

5.4.3.1.5.8    GetFAX

When the caller selects a key with this keyword as its choice,

ANSWER drops with a Fax Code errorlevel of 33.

5.4.3.1.5.9    GetCode

When the caller selects a key with this keyword as its choice,

ANSWER prompts the caller for a code of a specified length with

Tone 0.  If the code is valid, then the code is repeated back to

the caller ("NUMS.BIN").  Then "ICMEDIT.BIN" is played.  If no

Touch-Tone is received, then "TIMEOUT.BIN" is played.  If '0' is

received, "SAVED.BIN" is played and the code is recorded.  If '1'

is received, Tone 0 is sounded and the caller should re-enter the

code.  If '2' is pressed, the code is repeated back.  If '*' is

received, "EDITDONE" is played and this function returns.

5.4.3.1.5.10   GiveCode

When the caller selects a key with this keyword as its choice,

ANSWER generates and logs a unique code and says it out loud to

the caller.  A random code of a specified length is generated

consisting of digits from '0' to '9'.  This is repeated until the

code does not already appear in the GIVECODE.DAT (which is in the

PGM directory) text file.  The code is then saved to GIVECODE.DAT

file.  The configuration file is scanned for the keyword CodeDir.

If found, the directory name which follows it on the same line

after one or more spaces or tabs, as a filename equal to the code

(for example, 01234567.LOG).  If the keyword is not found, the

file is saved to the PGM directory.  The file will contain the

date and time followed by "Gave Code: nnnnnnnn".  The code will

then be spoken out loud to the caller.  Then "GIVECODE.BIN" will

be played, then Tone 3 sounded, giving the caller an option to

hear the code again.  Pressing '1' causes the code to be played

again.  Pressing '2', '*', '#', or '0' causes "SAVED.BIN" to be

played and the function to return.

5.4.3.1.5.11   VMail

When the caller selects a key with this keyword as its choice,

ANSWER switches to the Voice Mail Box mode.

5.4.3.2   TED Editor

Edits the TREE.CFG file.  The editor commands are listed below.

The keywords and configuration file format are described above in

"Edit Menu".

F1 : Exit editor without saving changes.

F2 : Undo the last deletion.

F3 : Print the text to the printer.

F4 : Mark off text for cut.

F5 : Cut marked off text.

F6 : Paste text that was marked and cut.

F7 : Exit and save changes.

F8 : Delete from cursor to end of line.

F9 : Find occurrence of string.

F10: Undelete Last.

ANSWER Voice Mail System           5 / Operation               34

5.4.3.3   Other Editor

(unavailable at this time)

5.4.4     PhoneBook Dialer

Edits phonebook entries.  There are 256 possible entries.  Each

entry contains the following editable fields.

Type Name Code ###-###-####   Script    Last Call          ____ ____ ____ ____________   ______    _________

5.4.4.1   Type

Type is VOICE, DATA, or FAX and determines how ANSWER should act

once the phone has been answered at the other end.  If the type

is VOICE, and the call is successful, then the Information Server

is invoked.  If the type is DATA and the call is successful, then

the Terminal Mode is invoked.  If the type is FAX and the call is

successful, ANSWER exits with the Fax Code errorlevel of 33.

5.4.4.2   Name

Name is an ASCII field identifying the entry.  This field is not

used by ANSWER, but is provided for your use to identify the

entry.

5.4.4.3   Code

Code is the dialing prefix code and refers to a dialing prefix

entered in the Dial Options.  This can be used to enter a dialing

prefix like 9, or 1-206-, or whatever is needed to completely

dial the number.

5.4.4.4   Number

###-###-#### is the telephone number.  Enter only the digits

required to complete the call.

5.4.4.5   Script and Last Called

Script and Last Called are not used at present.

5.4.4.6   Dial Options

This menu is used to enter long distance dialing codes.  Also,

you can enter your local area code here.

5.4.4.6.1 Enter dialing prefix codes.

These are codes used in the dialing directory which are used to

indicate that the number should be preceded with other numbers.

5.4.4.6.2 Enter redial pause delay length.

The 'Redial' key word tells ANSWER how many seconds to wait

between successive redials of a number in the outdialing mode.

The valid parameter range is 1 to 255 and is measured in seconds.

This defaults to 15.

ANSWER Voice Mail System           5 / Operation               35

5.4.4.6.3 Enter the null connect telephone number.

The 'NullPhone' key word is used during local message play and

record.  If a number is entered here, it will be dialed when

messages are played or recorded locally.  Some telephone

exchanges have a number which you can call for testing purposes

and will produce just silence.  If no number is entered here, a

'4' will be dialed every 'DialTime' seconds during play and

record.  This should be a seven digit number, but can be up to 29

ditits.  This defaults to no parameter (left blank).

5.4.4.6.4 Enter your local area code.

The 'LocalArea' key word is used for outgoing calls.  If the area

code of the number to be called equals this number, only the

remaining seven digits of the number to call will be dialed.

Valid parameter is a 3 digit telephone area code.  This defaults

to no parameter (left blank).

5.5  Help

5.5.1     Instant Help

This menu displays the ASCII text file called ANSWER.HLP.  You

may use the HOME, END, PAGE-UP, and PAGE-DOWN keys.

ANSWER Voice Mail System           5 / Operation               36

5.6  Options

5.6.1     Colors

You may customize your menu colors from this menu.  Use the

UP/DOWN ARROW keys to select the menu, the RIGHT/LEFT keys to

select the boxes/text within the current menu, and the SPACEBAR

to select the color for the current item.

5.6.2     Directory

Select the directories for Program, Outgoing messages, Incoming

messages, Temporary working directory, and Bin messages.  If the

specified directory does not exist, you will be asked if ANSWER

should create the directory.  All directories default to the

current directory when ANSWER is run for the first time.

5.6.2.1   Program

Tells ANSWER where to find its operating files.

5.6.2.2   InComing

Enter the directory in which to keep incoming messages (ICM*.BIN

and voice mail box messages).

5.6.2.3   OutGoing

Enter the directory in which to keep outgoing messages (OGM*.BIN

and Information Server voice messages).

5.6.2.4   Temporary

Enter the directory in which to write temporary files.

5.6.2.5   Bin

Enter the directory in which to keep the required operating voice

files (all pre-recorded voice files).

5.6.3     Hardware

5.6.3.1   Addresses

Select the base address / irq pair for the modem or VMC hardware.

Refer to the modem or VMC User's Manual for possible address and

interrupt settings.  See Chapter 4 Setup/Address/Table 1 for more

information on hardware address.

5.6.3.2   Mouse

Enables/disables and sets the mouse sensitivity.

ANSWER Voice Mail System           5 / Operation               37

5.6.3.3   Zero Level

Adjusts the level at which ANSWER decides there is no audio and

stops recording.  Press ESCAPE to abort.  Use the ARROW keys to

increase or decrease the level.  You should set the threshold

level shown as a decimal number in white to equal the noise level

shown as a decimal number in red.  Press RETURN to select the

current level and perform the Voice Test.  The voice test will

sample the noise level of the phone line and will display a

vertical white line to the left of the threshold level line.

Press any key when you're finished with the test.  You may want

to re-adjust the level and perform the test again.  When

finished, press ESCAPE.  This test does not apply to the ZyXEL

and ZOOM since they perform zero level detection internally.

5.6.4     Messages

Sets some message record/play options.

5.6.4.1   Limits

Sets the maximum number of seconds for incoming messages (ICMs).

5.6.4.2   Buffers

Set the size of RAM buffer to be used during message playback and

record.  When using the no interrupt method for messages, an

audible gap can be heard in the message while ANSWER reads or

writes to or from the hard disk.  Use this option to reduce the

amount of gap.

5.6.4.3   Copy

This options is used to tell ANSWER to copy voice messages to the

temporary directory before playing them.  This is useful on

slower systems when an audible gap is heard in the message when

it is being retrieved from the disk.

5.6.4.4   Support

There are four files required by ANSWER which are pre-recorded

since their generation is not simple.  The daring may attempt it

here.  This is not for the timid.  Make sure you have a backup of

the files before you proceed.  When you start, you will have 1/2

second for each spoken word.  Each word will be written to a

temporary file, then used to make the one you are recording.

Play the file after you record it and then decide if you want to

try again.

5.6.4.4.1      NUMS

This file contains the spoken numbers from 0 to 59.  Each spoken

number is one half second in duration.

5.6.4.4.2      DAYS

This file contains the spoken days from Sunday to Saturday.  Each

spoken day is one half second in duration.

ANSWER Voice Mail System           5 / Operation               38

5.6.4.4.3      MONTHS

This file contains the spoken months from January to December.

Each spoken month is one half second in duration.

5.6.4.4.4      DIGITS

This file is only required by VMCs that do not have a touch tone

generator (all supported modems have one).  It contains each of

the touch tone digits and each is one quarter second in duration.

5.6.5     Tones

ANSWER can be programmed to use eight different tones during its

operation.  In this menu, you can program the tones.  A treble

and bass clef are used to display the notes.  Each tone has eight

notes.  Use the ARROW keys to select the pitch of the notes, and

the plus "+" and minus "-" keys to select the duration of the

notes.  Use the page up "PG-UP" and page down "PG-DN" keys to

select the different tones.

These tone numbers are used throughout ANSWER as indicated.  The

other tone numbers are currently not used.

Programmable Tones:

TONE_RECORD    0    /* prompt user to record message   */

TONE_USER      1    /* user programmable tone          */

TONE_PROMPT    3    /* prompt user for touch tone      */

Non-Programmable Tones

TONE_SINGLE    11   /* gen purpose single 400hz tone   */

TONE_NOTICE    12   /* gen purpose single high tone    */

ANSWER Voice Mail System           6 / Voice Message Files     39

6.   Voice Message Files

If all voice capable hardware were created equal, so might their

voice file formats.  This however is not the case.  Several

different voice file formats are used by ANSWER.  The formats are

explained in this section, and some conversion utilities are

mentioned.

6.1  Formats

All messages used by ANSWER have a ".BIN" extension.  All valid

DOS filenames are accepted except those beginning with a dash '-'

character.

6.1.1     DigiPhone voice file format

The voice file contains straight voice data in unsigned

binary (Unipolar Complementary) format.  That is, a value of 0

translates to -127, 128 is 0, and 255 is 127.  A header is placed

at the beginning of the file to identify it as being compatible

with .VOC files.

6.1.2     CompuCom voice file format

The CompuCom voice file contains straight voice data in

signed (Bipolar) format.  That is, a value of 0 is 0, -127 is -

127, and +128 is +128.  A VOC header is placed at the beginning

of this file, however, the SoundBlaster utilities do not support

the Bipolar format.  However, some third party utilities do.

6.1.3     ZyXEL voice file format

The voice file contains a file header and the compressed

voice data.  The voice data is just the data received from U1496

Modem/Fax.  The header is a 16-byte data structure as follows.

Bytes 0 - 4 : title  -> "ZyXEL"

Byte  5     : 2

Byte  6 - 7 : reserved

Byte  8 - 9 : reserved

Bytes 10-11 : voice compress scheme,

0 - CELP

1 - 2 bits ADPCM

2 - 3 bits ADPCM

Bytes 12-13 : reserved

Bytes 14-15 : reserved

6.2.3     VOC Files (SoundBlaster)

Files with .VOC extensions are in the Creative Labs Voice File

Format.  this file is organized in two main blocks, the Header

Block and teh Data Block.  The Header Block contains an

identifier, the version number and a pointer to the start of the

Data Block.  The Data Block is divided into sub-blocks of various

types, such as Voice Data, Silence, Marker, ASCII text, Repeat

and End Repeat, Terminator and theExtended Block.  This extended

block - Block Type eight is meant for stereo or high speed voice

data.

ANSWER Voice Mail System           6 / Voice Message Files     40

Header Block

Offset (hex)

0 - 13   File Type Description.  The following mesasge is stored

here: "Creative Voice File", 1AH

14 - 15   Offset of Data Block from the start of voice file.

This word points to the Data Block.  It helps the

application program to locate the Data Block in case

the size of Header Block is changed.  For this version,

the value here is always 1AH.

16 - 17   Voice file format version number.  This version number

allows your program to identify different organization

formats of voice data file in case of future

enhancement.  Low Byte and High Byte are the minor and

major version number resptively.  Current version is

1.10.

18 - 19   Voice file identification code.  This code allows your

program to check that this file is a VOC file.  Its

content is the complement of the file format version

number then plus 1234H.  For the current version of

1.10, it is complement(010AH) + 1234H = 1129H.

Data Block

The Data Block is subdivided into multiple sub-blocks of data.

The first byte of each sub-block is called the Block Type.  It

indicates the type of data contained in the sub-block.  The next

three bytes is a 24-bit Block Length.  It is the number of bytes

in the sub-block excluding the Block Type and Block Length

fields.  Only the Terminator sub-block does not have the Block

Length field.  Depending on Block Type, the rest of the data in

the sub-block may be voice attributes (such as sampling rate,

packing and voice mode), voice data or other information (such as

Marker and ASCII text).  Program need not interpret all the Block

Types.  For those Block Types it does not recognize, it can

easily move to the next sub-block using Block Length.  In most

applications, the program can simply pass the address of the Data

Block to the Voice drivers which will interpret the Voice file

and perform the output accordingly.

6.3.2     WAV Files

Files with .WAV extensions are in the Microsoft Multimedia RIFF

WAVE PCM format.

6.2  Conversions

6.2.1     VCNVT.EXE

The ZyXEL ZFAX utility includes a voice conversion program which

will convert the ZyXEL voice files to VOC files.  The voice files

used by ANSWER are the 2 bit ADPCM type.

6.2.2     ZYX2WAV.EXE

This program converts ZyXEL sound files to Microsoft Mulimedia

RIFF WAVE PCM standard format.  The final .WAV file is PCM, mono,

ANSWER Voice Mail System           6 / Voice Message Files     41

9600 samples/second.  This is readable by Windows Sound Recorder.

This is Version 1.0 and should work with ADPCM2 or ADPCM3.  When

it's a little prettier the author will furnish source to anyone

who wants it.  The author plans to convert the other way soon.

Author:

Matt Galloway

423 S. Jefferson

Stillwater, OK 74074

mat@datacomm.ucc.okstate.edu

Usage : ZYX2WAV.EXE source  destination

source is filename of ZyXEL file

destination is filename of .WAV file

6.2.3     ZYXELVOC.EXE

ZyXEL Voice Data Conversion Utility

(c)  Norbert Igl, 10.10.92

! PUBLIC Beta Version 1.1 !

Features :

Converts between:

2-Bit-ADPCM ZyXEL --> 8-Bit-RAW SB-VOC, File = 1Noname.VOC

2-Bit-ADPCM ZyXEL --> 2-Bit-ADPCM SB-VOC, File = 2Noname.VOC

8-BIT-RAW SB-VOC  --> 8-BIT-RAW SB-VOC with 9600 Sampling

.. resulting File = "3Noname.VOC"

8-BIT-RAW SB-VOC  --> 2-Bit-ADPCM ZyXEL

( any Samp.Rate )    ( 9600 Samp.Rate )

.. resulting File = "0Noname.ZVD"

Plays VOC on Soundblaster

Plays 8-Bit-Voc on PC-Speaker ( if enough RAM ... )

Gives detailed Info about VOC and ZVD Files

".ZVD" .....

this is file-extension for  ZyxelVoiceData

-    -    -

Files :

ZYXELVOC.EXE Main Programm

VOC_SB.EXE   for SB-Output of VOC

VOC_PC.EXE   for PC_Speaker-Output of VOC

ZYXELVOC.VOC Hello-Voc (:-)

ZYXELVOC.CFG Config File, will be created if not present.

Please report any problems to 2:2402/300.3!

6.2.4     VOC2WAV.EXE

Converts Creative .VOC files to Microsoft .WAV files.

6.2.5     WAV2VOC.EXE

Converts Microsoft .WAV files to Creative .VOC files.

With these programs, existing Sound Blaster Voice files may now

be easily ported to the Microsoft Multimedia platform, and vice

versa.

6.3  Standalone Utilities

ANSWER Voice Mail System           6 / Voice Message Files     42

These utilities allow you to perform voice functions with your

voice mail card, sound card, or fax modem from the command line.

6.3.1     ANSPlay

Plays any recognized voice file to the voice hardware.

6.3.2     ANSRec

Records a specified voice file type from the hardware.

6.3.3     Receive Fax

The ZyXEL modem comes with a command line utility for receiving

Fax's.  However, it fails to work in some environments.  An

alternative is available:

REFAX.EXE

NewAge productions

jonny bergdahl

Lillgatan 34 B

S-554 51  J NK PING

Sweden

2:204/503@fidonet

9:463/101@virnet

NewAge BBS, +46-36-121323, 1200-16800 ZyX.

+46-36-121323 (Using ReFax)

7.   Programming Examples

ANSWER Voice Mail System                8 / Files              43

8.   Files.  These files are distributed with the software.

CONTENTS  DOC  This file, contents of archives and release notes

LICENSE   DOC  software license agreement

ANSFAX    EXE  full screen receive fax status display

ANSWER    EXE  the main Voice Mail Software

DIG_GRAF  EXE  Graphic editor for the Unsigned Binary voice files

CSP_GRAF  EXE  Graphic editor for Signed Binary voice files

ANSWER    HLP  the help text file

ANSWER    DOC  the complete documentation for ANSWER

OFFER     TXT  ordering information

BINFILES  TXT  a text file specifying what the .bin files should

say

TED       COM  tiny editor from PC Magazine

WAIT      COM  wait utility to wait a specified number of seconds

DIG_SPKR  COM  pulse width modulation to PC's speaker of .bin

files, Unsigned Binary voice files

CSP_SPKR  COM  pulse width modulation to PC's speaker of .bin

files, Signed Binary voice files

PLAYSPKR  DOC  explanation of playspkr.com

ANS_FAX   BAT  receive fax batch file called from answering

machine

ANSWERIT  BAT  sample batch file for using ANSWER as a BBS front

end

PAGEUP    BAT  send file batch file called from terminal mode

PAGEDN    BAT  receive file batch file called from terminal mode

RECVFAX   BAT  example batch file for to receive a fax

REMOTE    BAT  batch file called from remote mode

TREETEST  CFG  sample tree.cfg file

ANSWER requires a number of operating files.  The following files

should be maintained in the program directory (see

Options/Directory).  In addition to these files, ANSWER will also

record and play outgoing and incoming messages which should be

maintained in the outgoing and incoming message directories.  The

default filename extension given to message files is ".BIN" but

this is arbitrary and any extension is acceptable.  You may re-

record any of the .BIN files in your voice if you wish.  The

following files are operating files which are not distributed

with the archive.

OPTIONS   DAT       Data file containing options

PHONE     DAT       PhoneBook data file

TREE      CFG       Information Server configuration file

OUTDIAL   CFG       Outdialer configuration file

ANSWER    LOG       An updated file recording incoming calls

PHONE     LST       List of hone numbers for the outdialer

VMAIL     EXT       Voice Mail Box database

VMAIL     SYS       Voice Mail configuration file

VMAIL     MSG       Voice Mail message database

The message files listed in BINFILES.TXT are required for normal

operation.  You may re-record any of these files with the

exception of NUMS.BIN, MONTHS.BIN, and DAYS.BIN which are

specifically recorded such that each entry is exactly 1/2 second

long.

ANSWER Voice Mail System                8 / Files              44

BYE       BIN  Last message file for Information Server

DAYS      BIN  Days of the week starting with Sunday - each 1/2

second long

DIGITS    BIN  Touch Tones for the DigiPhone

EDITDONE  BIN  "Editing is complete"

EDITING   BIN  "After the tone press 1 to delete this message,

2 to play the message again,

3 to play the next message,

4 to play the previous message,

or the pound sign to end this editing session"

HI        BIN  Played when outdial goes through

HOLD      BIN  Played before paging the operator when outdialing

mode active

ICMEDIT   BIN  "Press 1 to redo,

2 to listen,

3 to append,

* to quit without saving,

or 0 to save and exit"

INVALID   BIN  "That choice is not offered"

MAINMENU  BIN  Main Menu message file for Information Server

MESSAGE   BIN  Default introduction file

MONTHS    BIN  Months starting with January, each 1/2 second long

MSGS      BIN  "Messages"

NUMS0     BIN  Numbers starting 0, ending 19 - each 1/2 second long

NUMS10    BIN  Numbers starting 0, ending 90 - each 1/2 second long

increments of 10 (0,10,20,30,40,50,60,70,80,90)

OGMABORT  BIN  "Message Aborted"

OVER99    BIN  "Over 99"

PAGE      BIN  "Paging operator to the telephone"

PLAYCODE  BIN  "Press 1 to hear again, otherwise press 0"

PROMPT    BIN  "Press a Touch-Tone for voice"

REMOTE    BIN  "Entering remote mode"

RMOPEN    BIN  "Press 1 to play messages,

5 to execute REMOTE.BAT,

# to erase messages,

or * to quit and exit"

SAVED     BIN  "Message saved"

SEEYA     BIN  Played if "0" is pressed during outdial

TAKEON    BIN  "The following message was taken on"

TIMEAM    BIN  "A.M."

TIMEOUT   BIN  "Please press a number for you choice"

TIMEPM    BIN  "P.M."

TONES     BIN  Audible beeps for prompts, etc.

VMDEF     BIN  Default outgoing message filename

VMEDIT    BIN  "After the tone enter ext you would like to edit"

VMEDRT    BIN  "After the tone, re-enter the extension you would

like to edit"

VMNOFWD   BIN  "There is no next message"

VMNOPREV  BIN  "There is no previous message"

VMOPEN    BIN  "Welcome to Voice Mail.  After the tone, enter the

extension you would like to reach"

VMPWORD   BIN  "After the tone, enter your password"

VMPWRT    BIN  "After the tone, re-enter your password"

VMRETRY   BIN  "After the tone, please re-enter the extension"

YOUHAVE   BIN  "You have"

ANSWER Voice Mail System                9 / Command Line       45

9.   Command Line Parameters

Each of the four modes of operation may be called directly

from the Command Line by the use of command line switches as

listed below:

ANSWER /A:  Invokes ANSWER directly into the Answering Machine.

ANSWER /V:  Invokes ANSWER directly into the Voice Mail Box.

ANSWER /I:  Invokes ANSWER directly into the Information Server.

ANSWER /P:  Invokes ANSWER directly into the PhoneBook Dialer.

When started with any of the above parameters, ANSWER enters

the specified mode and waits for calls just as it would if

selected from the ACTIVATE menu.  Once the transaction is

complete, ANSWER will hang up the phone and exit.

The above parameters without the slash character will cause

ANSWER to enter the specified mode as if a call had already been

detected and the phone already taken offhook.  The transaction

will continue as normal, the phone will be placed onhook, and

ANSWER will exit.  This is referred to as the Automatic Mode.

With the above parameters, ANSWER may be used as a front end

software for a BBS.  A batch file (ANSWERIT.BAT) has been

provided as an example. ANSWER can be set to prompt for a Touch-

Tone entry to specify whether it should process a voice call, a

data call, or a fax call.  In this mode of operation, ANSWER can

be set to play a voice message prompting for the Touch-Tone

entry.  After ANSWER detects the ring, answers the phone, plays

the prompt message, it will wait 5 seconds for a Touch-Tone.  If

received, ANSWER will process the voice call.  If no Touch-Tone

is received, ANSWER will send ATA to the modem to detect data

calls.  If it receives a CONNECT message from the modem, it will

drop with an errorlevel equal to the numeric result code of the

CONNECT.  See the modem User's manual for a listing of the

numeric result codes.

ANSWER Voice Mail System                10 / Return Codes      46

10.  Return Codes

struct result_code csp_result_code[] = { CompuCom result codes

{ 0,  "OK",                     },

{ 1,  "CONNECT 300",            },

{ 2,  "RING",                   },

{ 3,  "NO CARRIER",             },

{ 4,  "ERROR",                  },

{ 5,  "CONNECT 1200",           },

{ 6,  "NO DIALTONE",            },

{ 7,  "BUSY",                   },

{ 8,  "NO ANSWER",              },

{ 9,  "",                       },

{ 10,  "CONNECT 2400",          },

{ 11,  "CONNECT 4800",          },

{ 12,  "CONNECT 9600",          },

{ 13,  "DIALTONE",              },

{ 14,  "TIMEOUT",               },

{ 15,  "TOUCHTONE",             },

{ 16,  "VOICE",                 }, /* addition */

{ 17,  "CONNECT",               }, /* addition */

{ 18,  "",                      },

{ 19,  "",                      },

{ 20,  "CONNECT 300/REL",       },

{ 21,  "",                      },

{ 22,  "CONNECT 1200/REL",      },

{ 23,  "CONNECT 2400/REL",      },

{ 24,  "CONNECT 4800/REL",      },

{ 25,  "CONNECT 9600/REL"       } };

struct result_code zyx_result_code[] = {  ZyXEL result codes

{ 0,  "OK",                     },

{ 1,  "CONNECT",                },

{ 2,  "RING",                   },

{ 3,  "NO CARRIER",             },

{ 4,  "ERROR",                  },

{ 5,  "CONNECT 1200",           },

{ 6,  "NO DIAL TONE",           },

{ 7,  "BUSY",                   },

{ 8,  "NO ANSWER",              },

{ 9,  "RINGING",                },

{ 10,  "CONNECT 2400",          },

{ 11,  "CONNECT 4800",          },

{ 12,  "CONNECT 9600",          },

{ 13,  "",                      },

{ 14,  "CONNECT 19200",         },

{ 15,  "CONNECT 7200",          },

{ 16,  "CONNECT 12000",         },

{ 17,  "CONNECT 14400",         },

{ 18,  "CONNECT 16800",         },

{ 19,  "CONNECT 38400",         },

{ 20,  "CONNECT 57600",         },

{ 21,  "CONNECT 76800",         },

{ 22,  "",                      } };

ANSWER Voice Mail System                11 / Index             47

11.  Index

Activate                           Diagnostics

Operation  . . . . . . 24          Examine  . . . . . . . 24

Address Usage                      Dial

Hardware                           Phone  . . . . . . . . 20

Installation  . . 14     Dial Options

Addresses                               PhoneBook Dialer . . . 34

Hardware . . . . . . . 36     Dialer

Hardware                           Activate . . . . . . . 27

Installation  . . 14     DigiPhone

Modems . . . . . . . . 13          Hardware

ANSPlay                                      Installation  . . 13

Standalone                    DigiPhone voice file

Utilities . . . . 42               format

ANSRec                                  Formats  . . . . . . . 39

Standalone                    Directory

Utilities . . . . 42          Options  . . . . . . . 36

Answer Mode                        Distribution

Answering Machine  . . 29          Preliminary  . . . . .  4

Answering Machine                  Edit

Activate . . . . . . . 24          OutGoing . . . . . . . 23

Control  . . . . . . . 28     Edit Extension

Software                           Voice Mail Box . . . . 27

Installation  . . 17     Edit Extension No.

Answering the Phone                     Voice Mail Box . . . . 30

Answering Machine  . . 25     Edit Menu

Voice Mail Box . . . . 26          Information Server . . 30

Bin                                Examine

Directory  . . . . . . 36          Messages . . . . . . . 23

Buffers                            Extension Digits

Copy . . . . . . . . . 37          Voice Mail Box . . . . 29

Messages . . . . . . . 37     Fax

Bypassing Interrupts                    Answer Mode  . . . . . 25

Hardware                      Fax Code Errorlevel 33

Installation  . . 13          Answer Mode  . . . . . 25

Call Log                           FileList

InComing . . . . . . . 21          PhoneBook Dialer . . . 28

Code                               Files . . . . . . . . . . . 43

PhoneBook Dialer . . . 34     First Run

Colors                                  Software

Options  . . . . . . . 36               Installation  . . 16

Command Line  . . . . . . . 45     Flash

Activate . . . . . . . 45          Phone  . . . . . . . . 20

CompuCom voice file                Formats . . . . . . . . . . 39

format                        Voice Message Files  . 39

Voice File Formats . . 39     General

Contacting the Author                   Preliminary  . . . . .  4

Preliminary  . . . . .  9     Graphic Editor

Control                                 Examine  . . . . . . . 23

Operation  . . . . . . 28     Hardware

Conversions                             Options  . . . . . . . 36

Voice Message Files  . 40     Hardware Installation

Data                                    Setup  . . . . . . . . 12

Answer Mode  . . . . . 25     Help

Delete                                  Operation  . . . . . . 35

InComing . . . . . . . 22     InComing

OutGoing . . . . . . . 23          Directory  . . . . . . 36

ANSWER Voice Mail System                11 / Index             48

Messages . . . . . . . 21          Phone  . . . . . . . . 20

Incoming Message                   Operation . . . . . . . . . 20

Recording                Options

Answering Machine  . . 26          Operation  . . . . . . 36

Incoming Msgs                      Other Editor

Voice Mail Box . . . . 30          Information Server . . 34

Information Server                 OutGoing

Activate . . . . . . . 27          Directory  . . . . . . 36

Control  . . . . . . . 30          Messages . . . . . . . 22

Software                      Outgoing Message Delivery

Installation  . . 19          Answering Machine  . . 25

Inserting the voice mail           Page Operator

card                          Answering Machine  . . 25

Hardware                      Password Digits

Installation  . . 15          Voice Mail Box . . . . 30

Installing The Software            Phone

Software                           Operation  . . . . . . 20

Installation  . . 16     PhoneBook

Instant Help                            PhoneBook Dialer . . . 28

Help . . . . . . . . . 35     PhoneBook Dialer

Interrupts                              Activate . . . . . . . 27

Hardware                           Control  . . . . . . . 34

Installation  . . 13          Software

Modems . . . . . . . . 13               Installation  . . 19

Introduction  . . . . . . . 11     Play

Jacks                                   InComing . . . . . . . 21

Hardware                           Messages . . . . . . . 21

Installation  . . 15          OutGoing . . . . . . . 23

Last Called                        Preliminary . . . . . . . .  4

PhoneBook Dialer . . . 34     Program

License                                 Directory  . . . . . . 36

Preliminary  . . . . .  5     Programming Examples  . . . 42

Limits                             Prompt

Messages . . . . . . . 37          Answer Mode  . . . . . 25

Listen for Touch-Tone              Receive Fax

Answering Machine  . . 25          Standalone

Mail Box Number                              Utilities . . . . 42

Voice Mail Box . . . . 27     Record

Manual                                  InComing . . . . . . . 22

PhoneBook Dialer . . . 28          Messages . . . . . . . 21

Messages                                OutGoing . . . . . . . 22

Operation  . . . . . . 21     Remote Code

Options  . . . . . . . 37          Answering Machine  . . 29

Support  . . . . . . . 37     Remote Mode

Modems                                  Answering Machine  . . 26

Hardware                      Requirements  . . . . .  9, 10

Installation  . . 12     Return Codes  . . . . . . . 46

Mouse                              Rings

Hardware . . . . . . . 36          Answering Machine  . . 29

Name                               Script

PhoneBook Dialer . . . 34          PhoneBook Dialer . . . 34

Number                             Select

PhoneBook Dialer . . . 34          OutGoing . . . . . . . 22

Off Hook                           Session Completion

Phone  . . . . . . . . 20          Answering Machine  . . 26

OGM Number                         Setup . . . . . . . . . . . 12

Answering Machine  . . 28     Software Installation

On Hook                                 Setup  . . . . . . . . 16

ANSWER Voice Mail System                11 / Index             49

Spectrum Analyzer

Examine  . . . . . . . 23

Standalone Utilities

Voice Message Files  . 41

Support . . . . . . . . . . 37

DAYS . . . . . . . . . 37

DIGITS . . . . . . . . 38

MONTHS . . . . . . . . 38

NUMS . . . . . . . . . 37

TED Editor

Information Server . . 33

Temporary

Directory  . . . . . . 36

Terminal Mode

Phone  . . . . . . . . 20

Times/Dates

InComing . . . . . . . 22

Tones

Options  . . . . . . . 38

Type

PhoneBook Dialer . . . 34

User/Extension Edit

Voice Mail Box . . . . 30

VCNVT.EXE

Conversions  . . . . . 40

VOC Files (SoundBlaster)

Formats  . . . . . . . 39

Voice Mail Box

Activate . . . . . . . 26

Control  . . . . . . . 29

Software

Installation  . . 18

Voice Message Files . . . . 39

Volume Control

Hardware

Installation  . . 14

Wait for Calls

Answering Machine  . . 24

Voice Mail Box . . . . 26

WAV Files

Formats  . . . . . . . 40

Zero Level

Hardware . . . . . . . 37

ZYX2WAV.EXE

Conversions  . . . . . 40

ZyXEL voice file format

Formats  . . . . . . . 39

ZYXELVOC.EXE

Conversions  . . . . . 41