DISKLIST LASER COMMAND GUIDE

Revised December 3, 1993

This is a guide to help you use DISKLIST on a HP LaserJet.  Since

other brand laser printers emulate the HP PCL (Printer Control

Language) this information is applicable to most laser printers.

The purpose is to help you customize the laser commands that

DISKLIST sends to the printer.  This will give you control over

how and where DISKLIST is printed.

The basic syntax of a PCL command is:

The first character is ESC

ESC is followed by printable ASCII characters - letters,

numbers and symbols.

The last character is always an uppercase letter.

The simplest command is to reset the printer - ESC E.

There is a space after ESC in this documentation for clarity.  In

the actual HP laser command there is no space.  In the DISKLIST

laser command editor, the ESC is displayed as a left arrow with

no space.

ESC is usually followed by two characters that identify a family.

The first is usually a symbol and the second a lowercase letter.

These are followed by a number.  It can be an integer or a

decimal to 4 decimal places, depending on the command.

Example:

To set the point size to 12, the command is:   ESC (s12V

To set the pitch to 10 characters per inch:    ESC (s10H

Note that the integers match the desired value of the parameter,

and the point command ends in V for vertical and the pitch

command ends in H for horizontal.  Both commands have the family

characters (s.

One final rule.  Commands that have the same two characters after

ESC can be grouped together without repeating the ESC or the

common 2 characters.  In this case, the last letter of each

imbedded command must be lowercase.

-1-

Example:

Using the commands of the previous example to form one

command produces:

ESC (s12v10H

Note the v is now lowercase.

Warning:  Case is very important.  Also, do not confuse the

number 0 and the letter O, or the number 1 and the lower case L.

If you are not getting the results that you expect, look at your

commands very carefully for these substitutions.

LASER COMMAND SETUP

DISKLIST has a set of laser commands for each format.  They are

in the file DISKLIST.LSR.  It also has a laser command editor,

accessed with F6, so you can easily change them.  Remember to

save the changes you make in this file with F9.  The laser

commands are setup in the following categories:

Symbol

Spacing

Pitch & Point Size

Orientation

Top Margin

Lines per inch

Left Margin, Row

Reset

You can put any commands on any line.  These categories are for

reference only.  Although the order is important because the

value of one command effects the operation of other commands.

This will be discussed later with left margin and row.

When laser printer is selected, DISKLIST concatenates all of the

commands in the first 7 categories and sends them to the printer.

After the list is printed, DISKLIST sends the commands in the

eighth category, Reset.

You don't have to have any laser commands.  Many of these

parameters are inherent in the default font or can be set by

controls on the printer.  You may need a few commands, such as

top and left margin, and lines per inch.  The editor has a key

to delete the commands on one line, F7, or all commands for a

format, F8.

Laser commands are all printable ASCII characters except ESC.

The IBM character set displays ESC as a left pointing arrow.  So

does DISKLIST.  To enter ESC on the command line, just press the

-2-

ESC key.  Since the ESC key is used for input, press Function

Key 10 to exit the editor.  I try to make my software consistent,

so DISKLIST will quit all functions when F10 is pressed.

SYMBOL

Within a font set are many symbol sets.  The IBM lines that

DISKLIST uses to print the frames are in the symbol set called

IBM-PC(USA).  The command to use this symbol set is ESC (10U.

Some fonts do not have this symbol set, so the printer will

select another set close to it, but that symbol set probably

won't have the IBM lines.  In that case you can use the ASCII

lines or select a label format that doesn't have lines.

SPACING

This command sets the character spacing to proportional or fixed.

Since DISKLIST prints columns and a frame, the fixed spacing must

be selected.  The command for fixed spacing is ESC (s0P.  For

proportional spacing the command is ESC (s1P.

Most fonts only come in one type of spacing, proportional or

fixed so this command is probably ignored.  And, you don't

want to use a proportional font with DISKLIST.

PITCH AND POINT SIZE

These commands were used in the example above.  DISKLIST uses two

sets, one for normal and one for compressed.  They are:

Normal         ESC (s10.00h12.0V

Compressed     ESC (s16.66h8.5V

ORIENTATION

This command determines whether the list will be printed in the

Portrait or Landscape mode.  The commands are:

Portrait     ESC &l0O

Landscape    ESC &l1O

-3-

TOP MARGIN

This command positions the list from the top of the page.

Remember, with most lasers there is a border around the page

where the printer cannot physically print.  If you set it up to

print there, nothing will be printed in this area.  The command

for the number of lines down from the top is ESC &l#E where #

represents the number of lines.

LINES PER INCH

Lines per inch control the vertical spacing of the data lines.

There are two ways to select this.  Either select a specific

number of lines per inch, or select a fractional vertical motion

index.

The lines per inch command is ESC &l#D where # is the number of

lines per inch.  However, unlike most of the other commands, this

command only allows specific numbers.  They are 1, 2, 3, 4, 6, 8,

12, 16, 24, and 48.

Line spacing by vertical motion index command is ESC &l#C.

# represents the number of 1/48 inch increments and can be a

decimal.  The following table illustrates some useful values but

any value is acceptable by PCL.

Lines

per

Inch          #

6.0         8.00

6.5         7.38

7.0         6.86

7.5         6.40

8.0         6.00

8.5         5.65

9.0         5.33

9.5         5.05

10.0         4.80

10.5         4.57

11.0         4.36

11.5         4.17

12.0         4.00

LEFT MARGIN

This command positions the list from the left edge of the

page.  Before version 3.0, DISKLIST used the command ESC &a#L

for the column number to start printing in, where # represents

the column number.  This is important when you position the

-4-

list on labels.  The positioning of the left margin is based

on the current horizontal motion index which was set by the

pitch command.

DISKLIST now uses ESC &l0U to set the left margin because it

is required for the shade feature.  This is the PCL5 command

for Left (Long Edge) Offset Registration.

If your laser printer does not have PCL5 compatibility, then

you may change the left margin commands to ESC &a#L in all the

formats you expect to use.  You may edit the file DISKLIST.LSR

with the DOS EDIT and use Search and Change, or you may use

the DISKLIST.LSR file from an earlier version of DISKLIST.

This change will only work with the Output Devices Laser

Printer and Ink Jet Printer.  The laser commands for all the

other laser output device selectios are imbedded in DISKLIST.

SHADE will still work after this change if you set the left

margin to 0.

ROW

Row positions the start of the list down from the top margin,

not the top of the page.  The command is ESC &a#R where # is the

number of lines below the top margin.  Since it is in the &a

family, DISKLIST puts this in one command with the left margin.

The positioning of the row based on the current vertical motion

index which was set by the lines per inch command.

DEFAULT COMMANDS

F6 will insert these commands in the editor for the selected

format only:

Pitch ==>   Normal             Compressed

Symbol              ESC E ESC (10U        ESC E ESC (10U

Spacing             ESC (s0P              ESC (s0P

Pitch & Point Size  ESC (s10.00h12.0V     ESC (s16.66h8.5V

Orientation         ESC &l0O              ESC &l0O

Top Margin          ESC &l0E              ESC &l0E

Lines per inch      ESC &l6D              ESC &l8D

Left Margin, Row    ESC &l0U ESC &a1R     ESC &l0U ESC &a1R

Reset               ESC E                 ESC E

The symbol command is preceded by the ESC E, the reset command.

This forces a page eject in some printers, so you may want to

eliminate it.  If laser printer is selected, CTRL E will also

send a page eject from the control window.  If you want to print

more than one list on a sheet, remove the reset, then use CTRL E

to eject the page.

-5-

DOWNLOAD FONT

DISKLIST sends the following commands to the laser when it

downloads your font that you have selected from the Download

Window.

Reset                    ESC E

Font ID Assignment       ESC *c#D

The Font File

Font ID Assignment       ESC *c#D

Make Font Permanent      ESC *c5F

Select Font              ESC (#X

# is the font number.  DISKLIST generates a number by adding 70

to the sequence number of the font in the font selection window.

Except selection 10, which is assigned the number 70.  So the

range of font ID numbers that DISKLIST assigns is from 70 to 79.

You can then reselect a font with the command ESC (#X.  The

downloaded fonts are lost when the printer is turned off.

MORE

This is just a beginning of learning about PCL.  It is sufficient

to produce useful lists and labels.  There are many more commands

that may be of help in configuring DISKLIST to print lists

exactly as you like.  They are all in your printer manual.  I

hope this write-up helps you to read and understand the laser

printer manual.

HELPFUL HINTS

If you download or select a font, delete all laser commands

(use F8 in the editor), then insert a command for lines per

inch and commands for top, left margin, and row.  Otherwise

the commands will override the selected font and the printer

will select another.

If the vertical IBM lines look like dashes, squeeze the lines

together by printing more lines per inch.  Use the ESC &l#C

command.

With a laser printer you can create your own list formats.  For

example, if you have a 6 point 25 pitch font, you can use the

format for 5.25 inch 5 across to print a list that can hold 165

file names and be cut to fit the size of a 3.5 inch disk.  Then

tape the list to the back of the diskette with one small piece of

tape placed along the edge of the diskette that doesn't go into

the drive.  You then have a list that stays attached to the

diskette even when it is inserted in the drive.

-6-

DISKLIST LASER COMMAND REFERENCE CARD

Reset                   ESC E

Point Size              ESC (s#V

Pitch                   ESC (s#H

Portrait                ESC &l0O

Landscape               ESC &l1O

Top Margin              ESC &l#E

Left Margin             ESC &l0U  (PCL5)

Left Margin             ESC &a#L  (Before PCL5)

Row                     ESC &a#R

Lines Per Inch          ESC &l#D

Valid Values of # for lines per inch are:

1, 2, 3, 4, 6, 8, 12, 16, 24, and 48.

Vertical Motion Index   ESC &l#C

# represents the number of 1/48 inch increments and can be

a decimal.  The following table illustrates useful values:

Lines

per

Inch          #

6.0         8.00

6.5         7.38

7.0         6.86

7.5         6.40

8.0         6.00

8.5         5.65

9.0         5.33

9.5         5.05

10.0         4.80

10.5         4.57

11.0         4.36

11.5         4.17

12.0         4.00

-7-