-----------------------------------------------------

ACME:RealTime Spectrum Analyzer + Digital Osciloscope

-----------------------------------------------------

by Antonio J. Anton for Barcelona IEEE Student Branch

1. System needed:

ACME works on a 386SX+387 at 25MHz or better machine. Recommended 486DX-33

No memory requeriments (512Kb+)

ACME DOESN'T works with 387 emulators or under Windows. Use a plain DOS

ACME uses INT8 -> disk drives & timer are hold while running.

2. Keyboard

2.1 Spectrum Analyzer

Z: SWAP         : Switch to osciloscope

H: HELP         : Show a poor help window

F: FREQ.        : Select new sampling frequency

S: SAMP.        : Select number of samples to compute FFT (2**N)

I: IN FROM      : Redirect the input (LPT1, SoundBlaster, etc.)

W: WINDOW       : Select the window for input samples

M: MODE         : Show the output signal as |X[k]|**2, Re{X[k]}, etc

P: PLOT         : How to draw the output signal (Interpolated, dots, bars)

A: F AXIS       : Select a frequency range to display (lineal or dB)

Y: Y AXIS       : Select the magnitude axis (lineal or dB)

R: REFRESH      : Toggle persistence mode on/off

E: EXPR         : Enters the expression evaluator

O: OPTIONS      : Advanced options (PDF histogram, etc)

L: LOCATE       : Moveable axis over signal (for accurate measurements)

Q: QUIT         : Go to DOS

2.2 Osciloscope

Z : SWAP        : Switch to spectrum analyzer

H : HELP        : (like spectrum analizer)

I : IN FROM     : (idem)

F : FREQ        : (idem)

T : T AXIS      : Select a time range to display

Y : Y AXIS      : (like spectrum analyzer)

P : PLOT        : (idem)

R : REFRESH     : (idem)

S : SLOPE       : Set the trigger slope (ascendent, descendent) (*)

G : TRIGGER     : Set the trigger treshold (*)

O : OPTIONS     : (like spectrum analyzer)

E : EXPR        : (idem)

L : LOCATE      : (idem)

Q : QUIT        : (idem)

(*) Not yet implemented

2.3 LOCATE Option (in spectrum analyzer or osciloscope)

Q               : Go to main menu

A               : Toggle between axis #1 or #2

L               : Draw a line between axis #1 and #2

W               : Put text over actual axis position

Cursor Arrows   : Moves actual axis

[CTRL]+Arrows   : Moves actual axis (FAST)

3. Input signals (IN FROM option)

-ZERO   : Input is always zero (no signal)

-LPT1   : Input from LPT1 with ACMESAMPLER (a cheap A/D built by me)

-SB     : Sound Blaster (MIC or LINE jack). Note: I tested ACME only with

version 2.0 of S.B. I don't know if ACME works with others S.B.

The maximum sampling frequency of S.B. 2.0 is 11 KHz, if you try

an bigger number, the signal shown won't be correct.

-INT 81 : Thirdy-part samplers:

You must write a TSR that hooks int 0x81. ACME will call this

interrupt for initializing the device or to get a sample.

* Initializing the sampler:

In parameters:  AX = 0

BX = (1193180 / SamplingFrequency)

CX = # of samples for compute FFT

Out parameters: AX != 0 if succesfully initialitation of

your device

AX = 0  if could not initialize (ACME will

enter in ZERO INPUT mode)

You should preserve other registers

* Get a sample

In parameters:  AX = 1

Out parameters: AX = actual sample (in integer format:

1 sign bit + 15 magnitude bits)

You should preserve others registers

-EXPR   : Built-in expression generator for deterministic or pseudo-random

signals. To use this option: select IN FROM>EXPR then enter

a function of time. Example x(t)=sin(2*pi*1000*t)

EXPR supports:

-Math operators +, -, *, /, ^ (power)

-relational operators <, >. Example x(t)=t<0.1 will return

1 for t<0.1 and 0 for t>=0.1

-assignment operator =. (For use in main-menu EXPR option

to allocate variables. Example: eval> delta_phi=0.1)

To list all asigned variables use in main-menu EXPR the

function: vlist(). Example eval> vlist()

-parenthesis (,) to nest expressions of any level.

-functions of one argument: acos(.), asin(.), atan(.),

ceil(.), cos(.), delta(.), u(.), cosh(.), exp(.),

floor(.), ln(.), log(.), pow10(.), sin(.), sinh(.),

sqrt(.), tan(.), tanh(.), sinc(.), randomize(.),

sgn(.). Example. sgn(cos(2*pi*1000*t)) generates a

square wave with a frequency of 1000Hz.

-functions of two arguments: atan2(.,.), mod(.,.), pow(.,.)

-functions of four arguments: sum(,,,), prod(,,,).

Example: sum(2*k, k, 1, 3) equals to 2*1 + 2*2 + 2*3

Example: prod(cos(k*t), k, 1, 2) equals to cos(t)*cos(2*t)

-functions of undefined number of arguments: min(...),

max(...) this functions returns the minumum and maximum

value in the list. Example max(2,pi,-1,10) = 10

-predefined constants: pi = 3.1415....

e = 2.7182...

ans = last computed value

frequency = actual sampling frequency

delta_time = 1/frequency

-predefined variables:  t = time, used in IN FROM>EXPR

4. Sampling frequency considerations

The minimum sampling frequency is 100Hz. The maximum is 150KHz, but you

cant change these limits in the source ACME.C.

The sampling subroutine doesn't checks for overrun, that is, if you get a

too big sampling frequency, the system will crash.

Remember that with Sound Blaster (1.0, 2.0) the top sampling frequency is

11KHz, beyond this limit the output signal showed is incorrect scaled.

5. Other considerations

ACME is freeware, you can do with it all you want but give credits.

You can update ACME with new options (or correct any bug). If you like,

rewrite the screen subroutines to work with VGA (actual is CGA). Do not

use the BGI (is too slow!).

If you think your new & personal version of ACME is better than actual,

PLEASE GIVE ME A COPY!!! I will accept all variants of ACME.

--------------------------------------------------------

ACME: RealTime Spectrum Analyzer and Digital Osciloscope

Antonio J. Anton

Barcelona IEEE Student Branch (Phone. +34-3-4016821)

snail-mail:     E.T.S.E. Telecomunicaci

Jordi Girona Salgado s/n

08034 Barcelona

SPAIN

e-mail:         ieee@ieee.etsetb.upc.es

--------------------------------------------------------